Version 1.808.1

November 2024

Table of Contents

Introduction	1

Format of the User Guide for Dispatchers	1

User’s Environment	1

iNAP FAM Profile Management Access	1

Alternate Authentication (FAMAuth is unavailable)	2

WildCAD-E Roles	2

Application Security Splash Screen	2

HelpDesk Contact Information	2

Part I:  Home Page	2

Section 1: The Ribbon	3

Username and Center ID	4

My Profile	5

Save Layout	5

Clear Layout	5

Save Map Layout	6

Clear Map Layout	6

Switch Centers	6

Log Off	6

Section 2: Icons and Function Keys	7

Section 3: Hamburger (Pull-down Menu)	7

Part II: Daily Routines	8

Section 1: Whiteboard	8

To Add an Entry:	9

To Delete an Entry:	10

Section 2: Station Dispatch Sequence	10

Section 3: Foreign Resources	11

Section 4: Restore Archived Resources	12

Section 5: Rostering Authorization	12

Section 6: Rotations	13

Record Assignment Grid	13

Assignment History Grid	14

Section 7: Web Comments	14

Section 8: Contracts	14

Section 9: Set Response Level	16

Section 10: Reassign Resources	16

On the left side of the panel:	16

On the right side of the panel:	17

Section 11: Restore VOID Incidents	17

To restore a voided incident:	17

Section 12: Line Ups	18

To change a status on all resources:	18

To change a status of one resource:	19

Multi-Select Resources	20

Section 13: View IRWIN Incidents/Resources	21

Section 14: Clear Resource Groups	22

Part III: Opening Operations	23

Section 1: Map Icon (F5) / Map Tab	23

Function Key	23

Map Tab	24

Layers List	25

Select the Center Data	27

Select from either Incidents, Resource, WildCAD Data or Other Data	27

Plus, and Minus sign Icons (Upper Right Corner of the Map)	28

Recent Map	29

Clear Point	29

Dropped Point	29

Map Footer	30

Add layer from a linked Uniform Resource Location (URL)	32

Refresh Resources Layer	34

Add Layer from a Zipped Shapefile	34

Create Incident	36

Section 2: Search Bar and Find Panel	36

Using the Search Bar	36

Using the Find Panel	37

Lat/Lon:	38

UTM	39

T/R/S	40

Response Area	42

Finding a drop point using Bearing and Distance	42

Finding a drop point using Triangulation:	44

Place Name:	44

Mile Post:	45

Section 3: Using the Display Information	45

Section 4: Print Map Function	48

Section 5: Search Incident Panel (F2-or Search Icon)	49

Section 6: Resource Status Panel (F7 or Resource Icon)	51

To Status Resources:	51

To change Current Location:	51

Adding Comments about a Resource to Daily Log	54

Adding Comments about a Resource to Incident Log	55

Resource Current Dispatch Location	56

Creating a Groups	56

General Rules	57

Examples	57

Cancelling a Groups	58

WildShare	58

Starting Law enforcement (LE) incidents from Resource Panel	59

Resource Status Color Code	61

Resource Integration	61

IRWIN Observer	62

Section 7: Open Incident Panel (F8 or Open Incident Icon)	63

Method 1 – Incident Type, or Mine or Stage	63

Method 2 – Incident Type	64

Export	65

Section 8: Daily Log Panel (F12 or Daily Log Icon)	65

Add Log Entry	65

Editing or Viewing the Daily Log by either Incidents or Resources or Both	66

Highlight and Action	67

Part IV: Roster	69

Section 1: Rostering	69

Creating a Roster	70

Section 2: Picklist Setup	73

Part V. Incidents	76

Section 1: Search Incident Panel (F2 or Search Icon)	76

Section 2: New Incident (F9 or Incident Icon)	77

Create Incident	77

Section 3: Incident Panel and Tabs	80

Header Information – Upper Portion	80

IRWIN Integration	88

Location Tab (LOC)	90

Response Tab (RESP)	91

Comments Tab (CMNT)	96

Actions Tab (ACTNS)	96

Distance (DIST)	96

Frequency Tab (FREQ)	97

Incident Log (LOG)	98

Text/Email	99

Number (NUMBR)	100

Notifications (NOTIFY)	100

Contracts (CONT)	101

Fires (FIRES)	102

Incident Command Post (ICP)	104

Aircraft (AC)	105

Move Up (MU)	106

Incident Commander (IC)	107

Questions (FI - Wildfire or varies)	107

Fiscal (FISC)	107

IRWIN Status (IRWIN)	109

Conflict Status (CONF)	109

Incident within Incident Status (IWI)	111

Section 4: Escape Prescribed Fire. Post Fire Events and Out of Area Response	111

Prescribed Fire Escapes (FI – Prescribed Fire)	112

Post Fire BAER (FM – Emergency Stabilization) and Post Fire Rehab (FM – Fire Rehabilitation)	115

Fire Support (FM – Out of Area Response)	117

Section 5: Timers	120

Open Timers (F4 or Open Timer Icon)	120

Create New Timer	122

Sound	125

Section 6: Complexes	127

Create a Complex	127

Incident Name	129

Log and IRWIN Buttons	129

Adding an Incident to the Complex	129

FireCode	131

Releasing an Incident from the Complex	132

Section 7: Field Interrogation File (F3 or FI File Icon)	132

Field Interrogation File (FI File) panel	132

Searching by Resource (Block 1)	134

Searching by Name/DOB or Op License State/Number or Firearm	135

Part VI: Phone Directory	137

Section 1: Using the Phone Directory	137

To Refresh the Phone Directory	137

To Look Up a Person in the Phone Directory	137

To Clear the Search	137

Adding a person to the Phone Directory	137

Part VII: Text/Email	138

Section 1: Using Text/Email	138

Two Ways to Send Text/Email Messages	138

Select Recipient Name Text/Email Message	138

Compose Preset Messages	141

Selecting a Group Name Text/Email Message	141

Part VIII: Reports	142

Section 1: Available Reports	142

Summary Graphs/Reports	142

Normal Response Run Card	143

Daily Log Report	146

Morning Lineup Reports	148

In Service Report	149

Timer Report	149

Custom Incident Report	150

Part IX:  Links	153

Section 1: Websites and Documents	153

Websites	153

Documents	153

Part X: Appendices	155

Appendix I – Icons and Function Keys	155

Appendix II - Grids	157

Sort Order of a Grid	157

Add a Record or Row to the Grid	158

Delete a Row to the Grid	159

Actions and Settings at the Top of the Grid	159

Appendix III - Reports	162

Available Reports	162

Morning Lineup Reports	162

Responding Resource List	163

Open Incident	164

Incident By Date	164

Fire Report	165

Monthly Fire Summary	166

Contract Report	166

Appendix IV – Incident Types	167

Appendix V – Other Data Zoom Levels	169

Appendix VI – Other Data URL Layers	170

Part XI: Alternate Authentication (If FAMAuth is Unavailable)	170

Part XII: HelpDesk Contact Information	173

# Introduction

Welcome to WildCAD-E, the web-based Computer-Aided Dispatch (CAD) system developed by Bighorn Information Systems for wildland fire agencies and targeted for Emergency Communications Centers of all sizes.

The purpose of the WildCAD-E User Guide for Dispatchers is to lead a Dispatcher through a typical duty day.

## Format of the User Guide for Dispatchers

The WildCAD-E User Guide for Dispatchers is set up in multiple parts, with various associated sections described to lead the user through each part.

## User’s Environment

Considerations to Ensure Continuity in the Use of WildCAD-E and Workspace Suggestions for the User

- Dispatchers need a computer with either a Google Chrome or Microsoft Edge browser. Make sure the browser used is current.

- Field employees doing rostering can use a computer, tablet or phone.

- Two computer monitors available to WildCAD-E allow the dispatcher to drag items to a separate monitor.

- A cell-based hotspot is recommended for backup in the event the user loses internet connectivity.

- A fast internet is essential. Test your internet speed at https://www.speedtest.net.

- The session timeout of 8 hours has been removed and is being updated to the time the user logged out of WildCAD-E after 8 hours of inactivity in the application.

## iNAP FAM Profile Management Access

iNAP Org Unit Managers use their iNAP FAM Profile Management screen to:

- Complete steps the Org Unit Managers must take through iNAP to assign centers and roles.

- Approve or deny user request and to grant access, even when a user has not previously requested it.

- Approve access requests submitted by users and grant access to their dispatch center for users who have not requested access.

## Alternate Authentication (FAMAuth is unavailable)

Alternate Authentication (See Part XI) feature to allow users to log into WildCAD-E using an alternate secure method, if FAMAuth is experiencing an outage and is unavailable.

## WildCAD-E Roles

- Center Administrator provides access to the Center Admin menu.

- Dispatcher allows access to all dispatcher-related screens.

- Roster provides access to just rostering capabilities in WildCAD-E.

A dispatcher who will also roster will need both the Dispatcher role and the Roster role.

## Application Security Splash Screen

The user will now be presented with a page of security and appropriate use behavior information that must be agreed to before entering the application.

## HelpDesk Contact Information

If a user needs technical support from WildCAD-E, go to the WildCAD-E Support site at: http://www.wildcadsupport.net/login.asp; enter user’s name and password; and complete the online form. Support personnel will respond as soon as the form is received.

# Part I:  Home Page

The Home page is divided into three major sections: 1) the ribbon across the top, 2) the Icons, and 3) the pull-down menu or the “Hamburger” menu (≡), which displays the pull-down menu.

Figure 1- Home Page Ribbon shows System, Status of CAD, About, User and Dispatch Center (L-R)

Figure 2 - The Icons are located below the Ribbon.

Figure 3 - The "Hamburger" is located above and to the left of the Icons.

## Section 1: The Ribbon

The Ribbon includes:

- System – Indicates current operating environment and version.

- CAD Status – On or offline.

- About – By clicking on “About,” a pop-up window will open. Closing the pop-up will return the user to Home Page. Users can view the current version and system being used. (See Figure 4) Click on “WildCAD-E Release Notes” button to retrieve all the release notes. (See Figure 5)

- Help – Clicking on “Help” will take the user to WildCAD-E training.

- Username and Center ID – Indicates user, the current dispatch center, and where you can find the profile information for the current user. You can do this by clicking on the name (see Figure 6).

Figure 4 - Further defines what is available under “About.”

Figure 5 - Release Notes

### Username and Center ID

Figure 6 - Further defines the information available under "Username and Center ID."

### My Profile

Click on “My Profile” to display the following by clicking on the user’s email in the upper right corner:

Figure 7 - My Profile provides information about the User.

The “My Profile” option opens as a pop-up rather than opening on a new page. This allows the user to maintain the same content on the home page and not refresh when checking their profile.

Displays the attributes provided from iNAP and cannot be modified in WildCAD-E. Any changes will need to be made through the Center’s iNAP Org Unit Managers.

- My Information – Displays the user’s Email, Current Dispatch Center, and Current Unit that the user set as the Default.

- My Center Roles – This is a listing of what roles the user has, such as Center Administrator, Dispatcher and or Roster.

- Manage Map – By checking the box next to “Enable Map Caching for this Center” the shapefiles can be cached to improve map load time. If, however, you experience problems loading the map after turning this on, uncheck it and click "Clear Map Cache." This feature only applies to the users only for the center to which they are logged in.

### Save Layout

Use this menu item to save the current layout of user screens – which windows are open and where they are located – so that the same layout will appear at startup the next time the user launches WildCAD-E.

### Clear Layout

To remove the saved screen layout, use this menu item.

### Save Map Layout

Use this menu item to save the current map layout including zoom level, map center, and active layers. The save layer applies to both map panel and map in a separate tab – so that the same layout will appear at startup the next time the user launches Map.

### Clear Map Layout

To remove the saved map layout, use this menu item.

### Switch Centers

- Under “Switch Center,” use the dropdown menu to locate the appropriate dispatch center (Figure 8).

- Select the correct center from the list to change centers (Figure 9).

Figure 8 - Switch Dispatch Centers

Figure 9 - Select the new dispatch center from the dropdown menu.

### Log Off

- The action of logging off will require the user to log in as before.

## Section 2: Icons and Function Keys

Icons and Function Keys make up the second area of the Home Page and are reviewed in Appendix I – Icons and Function Keys. When a function key is available, users can either use the icon or function keys to access those panels within WildCAD-E.

## Section 3: Hamburger (Pull-down Menu)

Click the “Hamburger” symbol (≡) on the Ribbon, and the pull-down menu in Figure 10 will appear.

Figure 10 – Click on the ≡ (Hamburger) to access the pull-down menu.

Click on the appropriate topic in the pull-down menu (Figure 11), which will then allow the user access to each one of the menu items. Each menu item will open in its own “Tab,” except Links:

- Maps

- Phone Directory

- Text/Email

- Daily Routines

- Roster

- Reports

- Links

Figure 11 - Hamburger Pull-Down Menu

# Part II: Daily Routines

Figure 12 - Daily Routines open into a separate map.

## Section 1: Whiteboard

Figure 13 – To add an entry click on the plus sign and put the entry in a category.

The whiteboard is intended as a place to temporarily maintain information (for a day or two), it is NOT for long-term storage of information or a place to maintain a long history of any action or incident.

### To Add an Entry:

Figure 14 - Whiteboard Dropdown Menu.

To add a row, click the plus sign and enter the following:

- Category – use the Dropdown to select the category (See Figure 14)

- Initials – enter the dispatchers initials.

- Expires – enter the date and time this whiteboard entry will expire. Note; when whiteboard entries reach the expiration date, they will turn red.

- Entry – enter the message text.

- Display – the number entered will determine the message display order with the smallest number being displayed first.

- Click the “Save” icon.

Figure 15 – Click the “Save” Icon

### To Delete an Entry:

- Check the box for the entry to be deleted.

- Use the trash can to delete the entry selected.

## Section 2: Station Dispatch Sequence

Figure 16 - Station Dispatch Sequence is set by the Center Admin allowing resources to be dispatched in a preferred order.

Center Admin sets the Admin Defaults for the station dispatch sequence and does so for each resource type, allowing those resources to be dispatched in preferred order. Although the sequence is set, there may be times where resources are not dispatched according to the admin defaults.

Example of Re-sorting Station Dispatch Sequence. An example would be if Engine 311 is sequenced to go out before Engine 1R; but for that day, Engine 1R needs to go out before 311. To make that happen, assign a high number to Engine 311 and save it. The system will automatically re-sort the priority, so if only one engine is required for the day, Engine 1R will be dispatched first.

- Use Station Dispatch Sequence to set or reorder resource dispatch order by assigning the appropriate number in the sequence to the “Disp Seq” column.

- Identify the resource that should be top priority for the day.

- Assign a higher number to the current priority resource.

- The system will automatically re-sort the priority for that resource.

- To revert to the admin defaults, click the “Set to Admin Defaults” button.

## Section 3: Foreign Resources

Figure 17 - Foreign Resources are those resources temporarily assigned to the user’s assigned Dispatch area.

In WildCAD-E, the term “Foreign Resource” means resources that are temporarily assigned to the users assigned Dispatch area. Although only the WildCAD-E Center Admin can add or edit the users own Resources, all dispatchers may add/edit/Set Active (Yes/No) Foreign Resources as they come and go from the users assigned Dispatch area.

- To add a Foreign Resource, enter or select all required information, and click Save.

- Make any necessary edits and click “Save.”

- Code: Keep this as short as possible, since it will be displayed on numerous reports and lists. For example, use E31, not ENG31SQF.

- Type: Select from the dropdown the resource type (engines, dozers, etc.).

- Description Type: The description of the resource.

- Unit and Home Loc (Home Location) are all dropdown lists that the user created earlier.

- Lineup Seq (Line Up Sequence) controls the order in which Resources are listed on the morning Lineup Panel. This has nothing to do with the order in which they are dispatched - merely the appearance on the screen.

- Lineup Grp (Lineup Groups) are all dropdown lists that the user created earlier.

- Resource Cat/Type (Categories and Type) are all dropdown lists that the user created earlier.

- Current Location are all dropdown lists that the user created earlier.

- On Inc Rep Yes or No if the user wants this Resource listed on the printed Incident Reports.

- On WildWeb: Unless this is “Yes,” this Resource will not show on the internet reports from WildCAD-E called WildWeb.

- Active: Select “Yes” is the Foreign Resource still available. You cannot inactivate “No” resources while on an incident. Once a resource is inactivated, the resource will show the list of “Restore Archived Resources.”

- Comments: Enter text as it relates to this resource.

- F1 File: Select “No” is the Foreign Resource not authorized for law enforcement activities.

## Section 4: Restore Archived Resources

Figure 18 -Restore Archived Resources

Figure 19 - Restore resource by changing the "Active" column from no to yes.

- To restore an archived resource, go to the “Restore Archived Resources” screen, and change the “Active” status from no to yes.

## Section 5: Rostering Authorization

Figure 20 - Rostering Authorization

Rostering Authorization opens in its own Tab. Dispatchers can only authorize those personnel who have the Roster Role in WildCAD-E. If a Dispatcher is also going to do the actual rostering, the Dispatcher should have been assigned both the Dispatcher Role AND the Roster Role.

- Select the person’s name the user wants to have rostering authorization.

- Click the check boxes of which specific resources they will be authorized.

## Section 6: Rotations

The Center Admin sets up rotations at their respective center. Once rotations are set, they will appear here in WildCAD-E.

Figure 21 - Once rotations are set, they appear on the "Rotations" panel.

### Record Assignment Grid

- Data is filtered by selected Rotation Type, set by dropdown at the top of the page.

- Enter “Assign Date,” “Incident Number,” “Local Number,” “Assigned,” and “Release Date.” The columns are visualized to match rotation data.

- All columns are editable in the grid except for the “Sequence” and “Description” columns.

- Once a row has been edited, Click save, and the record will be added to rotation table.

Figure 22 - Record Assignment

### Assignment History Grid

- On this grid, the only entry allowed is the Release Date.

Figure 23 - Assignment History

## Section 7: Web Comments

Figure 24 - Web Comments are used to share any type of free text message.

Web Comments is an option for use by a dispatch center to share any type of free text messaging.

- Use the “Reset” button to display the last saved comment.

- Use the “Clear” button to clear contents.

- Enter comments and then click the “Save” button.

## Section 8: Contracts

Figure 25 - Contracts.

A dispatcher uses “Contracts” to manage the list of contractors for which the Center has agreements/ contracts for incident support. “Contracts” was developed before Viper, so many WildCAD centers now use Viper and no longer use the contracts concept in WildCAD. “Contracts” is where the dispatcher enters this information, so that the contracts will show up on the “Incident Contracts” tab. Dispatchers can add, edit and/or delete contract resource information here.

To add a contract resource:

- As with other grids, click the plus sign to add a new row.

- Use the dropdown to select the appropriate type of contract and any subtypes established by the users respective Center Admin.

- Complete the required information.

- Click the “Save” button.

NOTE: It is important to have the latitude and longitude for the incident because the LAT/LON will drive the proximity to the incident on the contracts tab.

Figure 26 - Contract Map shows location of Resources.

- No Action is required. These are informational screens. These screens allow the user to search for a specific resource to determine incident / location.

## Section 9: Set Response Level

Figure 27 -Set Response Level

- “Set Response Level” screen allows the user to set the daily response level for each resource by modifying the “Level” column.

- The date and time will automatically be modified to document the change.

## Section 10: Reassign Resources

Figure 28 - Reassign Resources

### On the left side of the panel:

- Select the “Source Incident” by either typing a few letters of the name or selecting from the dropdown list.

- Once the Incident is selected both the “Resource Code” (Name of Resource) and “Resource Status” will be populated.

### On the right side of the panel:

- Select the “Destination Incident;” and confirm reassignment of all resources from the old incident to the new, by selecting “OK.”

- Click “Re-assign Resource from Source to Destination.”

- All resources listed will be reassigned to Destination Incident as “Committed.”

Figure 29 - Confirmation of reassignment.

Figure 30 – Reassign resources from source to destination.

## Section 11: Restore VOID Incidents

### To restore a voided incident:

- Enter “Discovery Date” (From/To).

- Select the incident “Type” from the pull-down menu.

- Click “Search button or “Clear’ button to start over.

Search fields for date to and from and the incident type were added, as well as search and clear buttons. Since the page can only return up to 250 records, adding this feature will allow the user to search for any void incident no matter how old. If you exceed 250 records, an alert will appear.

Figure 31 - Restore VOID Incidents

After clicking the search button the list of incidents will appear.

Figure 32 - List of VOID Incidents

Select the appropriate incident by checking the box next to the VOID incident and use the “Restore” button to restore the incident.

Figure 33 - Restore VOID Incidents

## Section 12: Line Ups

### To change a status on all resources:

- Select the Lineup Group from the Lineups dropdown list.

- Click on either the “Out of Svc” or “Available” button. The user will be asked for confirmation.

Figure 34 - Line Ups Panel - All Resources

Figure 35 – Confirmation of Change to Out of Service

Figure 36 - Line Ups Panel - Out of Service

### To change a status of one resource:

- Select the Lineup Group from the Lineups dropdown list.

- Select the resource by either typing a few letters of the name or by selecting from the list. The status and location of the resource will be displayed.

- Select the appropriate status from the Resource Status dropdown list.

- Use the Resource Status dropdown list to change the status.

Figure 37 - Select the Resource

Figure 38 – Resource is Now Available

### Multi-Select Resources

- Select the Lineup Group from the Lineups dropdown list.

- Ctrl + Click on multiple resources.

- A red box will be displayed on selected resources

- Select the appropriate status from the Resource Status dropdown list.

Figure 39 - Selecting multiple resources.

Figure 40 - Selecting Multiple Resources

## Section 13: View IRWIN Incidents/Resources

This panel allows the users to view the data associated with WildCAD resources that have been matched Resources in IRWIN. Select a resource in the “Resource Code” column and the “IRWINRID” and associated information will be displayed to the right.

Figure 41 - View IRWIN Resources

The user can search for specific resource by enter the “Resource Code” in line below “Search by Resource Code.”

Figure 42 - Search for a Resource

Select an incident in the “Incident Name” column and the “IRWINID” and associated information will be displayed to the right.

Figure 43 - View IRWIN Incidents

The user can search for specific incident by enter the “Incident Name” in line below “Search by Incident Name.”

Figure 44 - Search for an IRWIN Incident by Incident Name

## Section 14: Clear Resource Groups

Use this menu item to clear (delete) all Resource Groups.

Figure 45 - Clear all Resource Groups button.

Figure 46 – “Clear All Resource Groups” button.

After using the “Clear All Resource Groups” button will indicate all groups have been cleared.

# Part III: Opening Operations

## Section 1: Map Icon (F5) / Map Tab

The Map can be opened in two ways:

- By selecting from the menu, the map will open in its own “Tab.”

- By clicking on the Map Icon button, the map will open on the “Home Page” screen.

The full screen map page has all the functionality of the map panel. In addition, it will show updated (live) incident and resource locations every three seconds.

When first launching WildCAD-E or when first opening the map panel, the map will take a second to load all the layers before it recenters the map on the extent shape layer identified by the center.

When the response area layer is refreshed for a center, the user is now alerted that a new file is available, and the user is instructed to refresh their map to see the new file. When a user logs in, the Response Area cache is refreshed if a new layer has been uploaded.

### Function Key

Figure 47– Map Icon (F5)

The Map Menu (F5) allows the user to view and utilize various GIS layers, which were integrated with WildCAD-E.

### Map Tab

Figure 48 - Map on Home page.

There are two (2) types of layers - Points and Polygons.

- Point data are shown as clustered icons on the map with a number showing how many points are in the cluster. These clusters will “explode” into smaller groups as the user zooms in or clicks on the cluster. Hover over a single icon on the map to show the identifying popup.

- Polygon data are displayed with different transparency and colors based on how the admin has set up the layer. Click inside the polygon to get information about it. It will either be displayed in the map footer, or the point info panel (described below).

### Layers List

Figure 49 - Layers List Icon

Figure 50 – Layers List has two sections—the Base Map Layers and Group of Data Layers

Clicking on the Layers Icon will display two sections.

- Upper Section – Base Map Layers

- Lower Section – Five groups of data from Layers List

The upper section allows the user to select one of the four map layers by selecting the radio button for which layer to be displayed. A horizontal scroll bars that is visible if the map panel size is smaller than the point info panel size.

Figure 51 - Base Map Layers

Bas

The lower section allows the user to select from five (5) groups of data in the layers list:

There are five (5) groups of data in the layers list:

- Incidents - These are points for open incidents broken out by incident types.

- Resources - These are points broken out by resource type.

  - Resources are displayed at the incident's location only when their status is Responding or On Scene or Available At Incident.

  - Resources with status "Out of Service", "Duty Officer Out Svc", "Unavailable for Assignment" and "Day Off" no longer display on the map.

  - Resources are displayed at their current dispatch location in all other cases.

  - Resource's home dispatch location and current dispatch location can be the same.

- WildCAD Data – Examples of these layers are:

  - Dispatch Location

  - Response Areas

  - Hazards are points, shown on the map as a large circle representing the hazard radius. Turn on the layer and hover over the circle to show the hazard name and the alert miles.

- Other Data - These are points and/or polygon data that have been added by the WildCAD Admin. Be advised if the EGP site is not available or there is a token issue. The map skips those layers and finishes loading rather than continually trying to load.

### Select the Center Data

To view the data (points or polygons) click on the “Plus” sign next to Center Data, then select from the layers group checkboxes.

Figure 52 - Lower Section - Five groups of data from Layers list.

The Center Data layers that are shapefiles are downloaded once they are added to the map, instead of when the map is opened. This prevents a delay in the map being functional when it is first opened.

In addition, on the Map Footer there is a checkbox “Identify Custom Layers?” When this checkbox is selected, information from the Center Data will be displayed, even if the Center Data layers are not checked.

The Center Data Layers:

- Have a “display field.” This is set up by the Center Admin.

- Are listed in the layers list under “Center Data.”

- May or may not be displayed on the map.

When the user checks the “Identify Custom Layers” box, a message “Data Loading,” will appear. This message should eliminate any confusion on whether the checkbox is working.

### Select from either Incidents, Resource, WildCAD Data or Other Data

The workflow is the same as Center Data. Below is an example using the Resources data:

Figure 53 - Map Feature Resource Locations assist the user in locating resource types.

By checking the boxes, this will display the resource types and their locations on the map; and if the user hovers over them, it will display the actual resource names. The number next to the resource indicates how many resources are at the location.

Figure 54 – Map displays location of resources.

### Plus, and Minus sign Icons (Upper Right Corner of the Map)

The “+ “and “- “sign icons are used for zooming map in or out. The mouse scroll wheel can also be used for zooming the map in or out.

### Recent Map

Figure 55 - This icon is to zoom the map back to starting position.

This will zoom the map back to the starting position, which is the extent of the dispatch center’s response areas.

### Clear Point

Figure 56 - Use of the "x" icon clears the dropped point from the map.

This clears the dropped point from the map, which was created when clicking the map. This also closes the map footer and the point info panel and clears the fields in the search panel.

### Dropped Point

Figure 57 - This is the dropped point icon.

Figure 58 - A point can be added to a map in several ways.

A point can be added to the map in several ways:

- Click anywhere on the map.

- Use the Search Bar (Enter Address, Place or Lat/Long)

- Use the Search Panel (Various search options such as, Lat/Long, TRS, Response Areas, Place Names, etc.)

The map footer will show information about the point. The point can be dragged to a new location and the map footer info will update.

### Map Footer

Anytime the user turns on the layer and hovers over a single icon or polygon on the map to show the description of that layer and some of which will be shown on map footer.

Single clicking anywhere in the map will “drop a point.” This action will open the “Map Footer,” which contains information about where the point is located.

Figure 59 - Map shows footer information.

The information fields are:

- Lat/Lon

- Response Area

- County

- TRS (township/range/section) TRS returned from the BLM service with no values will show in the footer as “N/A”.

- Meridian

- UTM Zone (eastings, northings, zone, hemisphere)

- The Current Weather Conditions include a general description of the weather, the temperature in degrees Fahrenheit, and wind speed (mph) and direction (cardinal direction). This information is from a MesoWest Weather Station link for the closest RAWS station at the dropped point and return observations within the last 24 hours of current day/time.

- Elevation.

- The Wilderness Names.

- The Predictive Service Area.

In addition, on the Map Footer there is a checkbox “Identify Custom Layers?” When selected, information from the Center Data will be displayed. The Center Data Layers:

- Have a “display field.” This is set up by the Center Admin.

- Are listed in the layers list under “Center Data.”

Map Services: If one of the mapping services that is used in the map footer is down or unresponsive, the footer will display “ERROR” for that layer’s data.

Figure 60 – Example a down or unresponsive layer

The WildCAD Admin can set that layer to be inactive. In the case where the layer is set to inactive, the footer will display “SRVC ERROR” for that layer.

Figure 61 - Example an inactive layer

Figure 62 - Zoom Level

Zoom Level

Zoom level value is now displayed in the bottom left portion of the maps so the user can easily see the zoom level of the map.

- The plus (“+”) and negative (“-”) sign icons are used for zooming the map in and/or out.

- At zoom level zero (0), the base map will display the entire globe.

- Each click of the plus (“+”) sign will increase the zoom level by one (1) zoom level, and one click of the negative (“-”) sign will decrease the zoom level by one.

When first opening the map, the zoom level will be automatically set to your predetermined extend zoom, usually the response area.

In the Maps – Other Data the zoom levels are predetermined and cannot be changed. In Appendix V will display the Other Data zoom level and other information.

### Add layer from a linked Uniform Resource Location (URL)

Figure 63 - URL icon.

Dispatchers can add URL data to their map from various sources such as map services hosted by ESRI. This data will not persist when the page is refreshed and is not visible on anyone else’s WildCAD map.

Enter the map URL and a name to display in the user’s layers list.

Figure 64 - Box used to enter Map URL and Map name to display in the User’s Layer.

If the URL is invalid the following will be displayed

Figure 63 – Invalid URL

The user will also see the file added to the layers list under a new heading “Your Data.”

Figure 65 - Your Data

Figure 66 - File will be added to the layers list.

### Refresh Resources Layer

Figure 67 - Refresh Resource Layer Icon

The “Refresh Resources” button will update the current location of the resources, which can be seen when the Resource Layers are turned on in the map. This avoids the need to refresh the entire home page or map page to get the latest location of resources.

Add Layer from a Zipped Shapefile

Figure 68 - Uploading Icon

Figure 69- Dispatchers can add data by uploading a zipped shape file from their computer.

Figure 70 - The zipped file must have four file extensions included:  .shp, .shx, .dbf, and prj.

Figure 71 - Once completed, the user will see the filed added to the map layers under "Your Data."

Dispatchers can add data to their map by uploading a zipped shapefile from their computer. This data will not persist when the page is refreshed and is not visible on anyone else’s WildCAD map.

Click the upload icon. A file browser will open. Locate the zipped shapefile, it must have at least these four (4) files extensions .shp, .shx, .dbf, and prj.

After selecting the file, it will be added to the user’s map, and the map will zoom to the file’s extent. The user will also see the file added to the layers list under a new heading “Your Data.”

### Create Incident

Figure 72 - Create a new incident icon from the map.

Incidents can be created directly from the map. After dropping a point on the map, click the new incident button. If using the map panel, this will open a new incident panel with the location information pre-populated.

Incidents can be created in the eastern hemisphere (i.e. Guam incidents). This can only be done by creating an incident from the map, or by dropping a point and clicking the “Use Map” button on the incident panel. Eastern longitudes and southern latitudes cannot be manually entered into the incident panel coordinate boxes.

## Section 2: Search Bar and Find Panel

### Using the Search Bar

Figure 73 - Search Bar

The Search Bar is a Google powered address locator. Search for addresses, place names or Latitude/Longitude. For example, type Joshua tree and the park will be located.

Figure 74 - Use the search bar to locate a specific area on the map.

Select the suggested search results. The address bar will reset, the map will zoom to that location, drop a point, and display information about the location in the map footer.

Figure 75 - Drop point is added to map for the location the user selects from the dropdown menu.

The user can also search by typing coordinates. For example: “35.122018, -119.549866.”

Figure 76 - Option to use Lat/Lon in search bar.

### Using the Find Panel

Figure 77 - Search icon.

The Find Panel has a vertical scroll bar that is visible if the map panel size is smaller than the find panel size. This allows the user to easily get to find panel functions without resizing the entire map panel.

When searching for response areas the code in the table will be matched to the code in the map regardless of if the case (upper/lower) is different between the 2 values.

UTM can use a default zone if set by the center administrator.

Figure 78 – If valid search, map will zoom to location and drop a point; the map footer will open and display relevant information.

Upon entering data into a new search row, the previous search criteria will be cleared.

### Lat/Lon:

- In the search panel, you can now enter Lat/Lon in D,M,S and D,dM format and tab over without it changing the format.

- In addition, the user can enter a space as well as a comma between degrees, minutes and seconds for latitude or longitude search.

- Then, click “Go,” and you will be taken to that drop point on the map.

Figure 79 – Entering the latitude and longitude.

### UTM

In the search panel, you can now enter Eastings, Northings and Zone. Then, click “Go,” and you will be taken to that drop point on the map.

Figure 80 - Enter UTM

### T/R/S

Enter the Township (T), Range (R) and Section (S) and then click “Go.”

Make sure the correct state and meridian are selected from dropdown to the TRS search. State and meridian default values are set by Center Admin are used or the user can select a value.

Figure 81 - Correct State and Meridian

General consideration while searching for T/R/S:

- Half townships and ranges now return the correct T/R/S.

- Find function returns a T/R/S when searching outside of the dispatch center's associated state.

- That PB stands for 'Protracted Block,’ which is an unsurvey area of which parts of, but not the entire, boundary has been surveyed.

  - A protracted block is identified by its unique numbering system. The lowest protracted block number will be PB37.

  - Map users should be aware of this difference in Section numbering from the standard 1-36.

  - The old section number is no longer the official identification number for that piece of land until the section is completely surveyed and accepted by the BLM.

Searching for an old section number (T4S/R26E/S36) that is a Protracted Block, WildCAD-E will show “no record found”.

Searching for a section that does not have a matching section in the BLM service, now returns a dropped point at the approximate center of the requested section. In this case, there is no section polygon to display on the map, (example of issue is 31N 51W 15, Nebraska, 6th Meridian).

Figure 82 - Searching for Section that is in Protracted Block.

Searching with a Protracted Block number (Section 51) and the same T/R, WildCAD-E will show “Old Section Number.”

Figure 83 - Searching for Section that is in Protracted Block Section Number (51)

### Response Area

Start typing the response area; and then, select from the dropdown or just click on the dropdown and select the response area. Then click “Go.”

Figure 84 – Searching for a Response Area

### Finding a drop point using Bearing and Distance

- Start typing “Location 1” and then select from the dropdown or just click on the dropdown and select the known location.

- Enter the Bearing and Distance.

  - Bearing and Distance labels change to indicate calculation after the user selects a location.

  - IF the location selected is designated a "Lookout", the label for Bearing is "(true)" and Distance is "(mi)" indicating the calculation uses true north and statute miles.

  - If the location selected is NOT designated as a "Lookout", the label for Bearing is "(mag)" and Distance is "(nm)" indicating the calculation uses magnetic north and nautical miles.

  - Then click “Go.”

Figure 85 – Lookout Bearing and Distance

Figure 86 - Aircraft Bearing and Distance

### Finding a drop point using Triangulation:

- Start typing “Location 1” and then select from the dropdown or just click on the dropdown, select the known location, and enter the Bearing and Distance.

- Start typing “Location 2” and then select from the dropdown or just click on the dropdown, select the known location, and enter the Bearing.

- Then at “Location 2” click “Go.”

Figure 87 - Triangulation.

### Place Name:

Searching for a Place Name

- Centers that have large lists of place names will now see their place names in the dropdown on the map search panel.

- Select the Place name click “Go" to zoom to the location.

Figure 88 - Place Names

Adding New Place Name

- Click the location of the new place name on the map.

Figure 89 - Drop Point on the Map

- Enter the name for the new place name and click the "ADD" button.

Figure 90 - Enter the name for this drop point.

### Mile Post:

Centers that have Mile Posts will now see their mile post in the dropdown on the map search panel and can select & hit "Go" to zoom to the location.

Figure 91 - Mile Post

## Section 3: Using the Display Information

Figure 92 - Display Information Icon

With the Center Data layer turned on, the point info panel will display the attribute information for each Center Data layer that the dropped point falls within. If an additional layer is turned on which encompasses the point, their attribute information will be automatically displayed.

The layers listed:

- Are layers listed under “Center Data.”

- May or may not have a “display field.”

- Must be turned “On.”

As Example,

- The “Counties” layer under “Center Data” is turned on - Figure 93.

- A drop point falls within this layer - Figure 94.

- Click on the ‘I” Icon to display every attribute from this layer – Figure 95.

Turning off a layer will remove it from the point info panel. Clearing the point using the “X” button on the right will remove the data grids in the panel, close the map footer and remove the point.

Figure 93 – Center Data – Select a Layer

Figure 94 – Center Data – Drop Point on Map

Figure 95 - Center Data – Click “I” Icon

## Section 4: Print Map Function

The Print Map button is in the upper left corner of the map. This button will produce a .PDF of the current map view with any layers that are turned on. Layer legend and map footer (where applicable) are displayed below the map.

Figure 96 Click on the Print Map Button

After clicking on the Map Button, the user will give the Print Map a file name.

Figure 97 Enter the File Name

Figure 98-Dispatchers Select the PDF file from their computer.

Figure 99 Example of a Print Map

## Section 5: Search Incident Panel (F2-or Search Icon)

Figure 100 - F2 or Search Icon

Each row in the search panel (Figure 98) represents one method to search for an incident.

As example,

- Enter a date from and to and what the user wants to search for on the incident.

- Click the Search button to execute the search. If the search was valid, the search will return a list of incidents (Figure 102).

To clear an enter, Click the “Clear” button.

Figure 101 Search Incidents using date only.

If search results exceed 250 records, the user is promoted to further limit the search to see all records.

Figure 102 - Search results by date ONLY 243 of 416 were returned.

Figure 103 - Search results were reduced to 56 records.

Search results by dates was reduced, and only 56 records were returned.

## Section 6: Resource Status Panel (F7 or Resource Icon)

Figure 104 - Resource Icon

On the Resource Status Panel, the user can set the resource status, location and enter comments. The “Assigned Incident,” will be populated once the resource is an incident, and this field is not editable from this panel.

### To Status Resources:

- Click on the Resource box. (The resource box has a black border).

- Using the pull-down under “Resource Status,” select a status.

### To change Current Location:

- Click on the Resource box. (The resource box has a black border).

- Using the pull-down under “Current Location,” select a status.

Figure 105 - Resource Status Panel.

The Resource Status Panel shows all resources. By using the pull-down functions, the panel can be sorted in four ways.

Sort by:

- Unit (Figure 106)

- Unit and Resource Type (Figure 107) or just by Unit

- Unit, Resource Type and Lineup (Figure 108) or just by Lineup

- Search by individual resources (Figure 109) name (“Code”).

Figure 106 - By Unit

Figure 107 – By Unit and Resource Type

Figure 108 - By Unit and Resource Type by Lineup

Figure 109 - Resource by Name ("Code")

For example, if the user searches by name (E44LPF), the select resource display shows:

- Resource Status “Avail Qtrs.”

- Current Location “OJAI”

- Resource ID, Home Location, Date/Time

Figure 110 - Select E44LPF

AND if the resource were assigned to an Incident, the name of Incident would be displayed.

### Adding Comments about a Resource to Daily Log

- Enter the Comment in free text.

- Change the Resource Status or click “Save.

Figure 111 - Example of entering a comment.

The comment will be displayed on the Daily Log (Figure 112).

Figure 112 - Example of entry on the Daily Log.

### Adding Comments about a Resource to Incident Log

- On a Resource that is assigned to Incident, E44LPF is assigned to Incident "B3-853."

- Added comment will appear in the Incident Log Tab.

- Incident B3-853 can be opened from this screen.

Figure 113 - Example E44LPF Committed Incident B3-853.

Figure 114 - Example of entering on the Incident Log.

### Resource Current Dispatch Location

Implemented business rules to properly set a resources current dispatch location based on their status:

- If their status is “Responding, On Scene or Available at Incident,” then the resource's current dispatch location is the dispatch location associated with the incident.

- If their status is “Returning to or Available in Quarters,” then, the resource's current dispatch location is the same as their Home Dispatch Location.

### Creating a Groups

- Click the “New Group" button to start selecting multiple resources for a group.

- Hold the Ctrl key down to select multiple resources.

- The selected resources will be outlined in RED.

- Click the “Save Group" button to save the resources selected for a new group.

Figure 115 - New Group

Figure 116 - Save Group

### General Rules

- Status and location changes made to any member of a group will apply to the entire group.

- When a comment is entered for a resource that is part of a group, that comment log entry will be applied to all resources in the group.

- When a resource's location is changed for a resource that is part of a group, the location change log entry will be applied to all resources in the group.

### Examples

When assigning any part of a group is selected a pop-up asked:

- Send the entire group – this sends all resources in the group.

- Send just selected resource – this just sends the selected resource and removes this resource from the group.

- Cancel – the individual or all will not be sent.

Figure 117 - Assign Group pop-up.

- If you have three resources in a group all can be assigned to an incident.

- If only one of three resources is assigned to the incident the group will remain with two resources.

- If only one resource is left in the group, then the group will be cancelled automatically.

### Cancelling a Groups

- Select one or more of the resources. The resources in the group will have a “Blue Box” around them.

- Click the “Cancel Group” button. This cancels the entire group.

Figure 118 - Cancel Group

### WildShare

Allows any center to make the status of one or more resources available to other centers. In turn, a center can view the status of another center’s shared resources

The Center Administrator for CALPCC is sharing (WildShare) Engine E371LPF by setting “YES” in the WildShare field.

Figure 119 – Allowing a resource to be shared

After the Center Administrator set in Viewing Center panel by selecting CALPCC resources, the Dispatcher on CASBCC will from the Resource Status panel, will click on WildShare button that opens another panel showing dispatch center that have shared resources.

Figure 120 – WildShare button on the Resource panel

The WildShare panel will open listing the center that have been shared. Click the down arrow to view the shared resources.  WildShare panel can be sized and positioned and then saved as part of the users saved layout.

Figure 121 – WildShare panel

Select the center to view what resources have been shared.

Figure 122 – Resource availability

### Starting Law enforcement (LE) incidents from Resource Panel

This feature can only be used if the center and Dispatchers have been LE Authorized. Unless the dispatch is authorized and "LE Authorized” is set to "Yes," the dispatcher will not have access to navigate to and use the FI File panel.

- Select the resource to be committed to LE Incident.

- Click the “New LE” button.

- A new incident panel will open (Figure 123).

Figure 123 - Committing a Resource to a new LE Incident.

Figure 124 - LE Incident Panel

### Resource Status Color Code

This code cannot be changed and is set by the government.

Figure 125 - Resource Status Color Codes.

### Resource Integration

Resource integration means exchanging information about WildCAD-E resources, such as engines, crews, helicopters, etc., and the people staffing them via IRWIN with other systems like IROC, IQS, and IQCS. These five systems have worked closely over the past three years to develop standards and methodologies for exchanging WildCAD-E resource information.

WildCAD-E passes resource statuses to IRWIN where other connected systems can read them. WildCAD-E automatically creates filled requests for local resources sent to local fires and experience will automatically be generated for people staffing the resources. Dispatchers will still use IROC for other tasks such as placing or filling unfilled requests.

When a dispatch action closes a filled request (e.g., set the resource's status Avail), the last step is to set the resource's General Status to Available.

When a resource that is on a filled request is reassigned from one incident to another, the current request will always be closed first before creating a new request.

WildCAD-E will block creating capability requests if the resource's Current Dispatch Unit is different from the unit of the user's dispatch center.

Resource's status values are no longer set in IRWIN when the resources are assigned to NON-IRWIN incidents.

Users may create a new request for a resource sent to their center on a preposition.

A resource's primary capability is read directly from IRWIN when the resource is assigned rather than only when the resource is matched between IRWIN and WildCAD-E. This solves issues where the resource's primary capability may have changed after the WildCAD-E resource was matched to the IRWIN resource.

### IRWIN Observer

In IRWIN Observer the resource request will appear once a resource is responding – provided the resource has been IRWIN Matched in WildCAD-E and the incident has been sent to IRWIN.

IRWIN Observer will show the operational status as Mob En Route when the resource is responding, will show the ETD as current date and time, and will show the ETA as current date and time plus 20 years. WildCAD-E uses plus 20 years so that IROC’s “travel job” does not automatically change resource statuses.

Figure 126 –The Resource is Responding – ETA set plus 20 years.

IRWIN Observer will show the operational status as Demo En Route when the resource is returning, will show DemobETD as current date and time, and will show the DemobETA   current date and time plus 20 years.

Figure 127 –The Resource is Returning– DemobETA set plus 20 years.

IRWIN Observer will show the resource request Closed and the DemobETA is set to the actual date and time the resource status was set to any status not on the current incident.

Figure 128 –When Request is closed - DemobETA and set the current date and time.

## Section 7: Open Incident Panel (F8 or Open Incident Icon)

Figure 129 - F8 Open Incident Panel

There are two methods for sorting open incidents.

- By clicking the radio buttons next to Type or Mine or Stage.

- By clicking the radio buttons next to Type or Mine or Stage and selecting an incident “Type” from dropdown.

### Method 1 – Incident Type, or Mine or Stage

On the Open Incident panel there are three main ways to sort open incidents by clicking on one of the radio buttons. Each of the radio buttons will retrieve all “Incident Types” based on the radio button selected.

As example,

- Type - will retrieve all open incident regardless of which dispatcher is associated with that incident.

- Mine - will retrieve all open incidents that are associated with an individual dispatcher.

- Stage – is like “Type” and will retrieve all open incident but will color code to represent their stage.

- Red - Not Contained

- Blue – Contained

- Green – Controlled

- Black – Not a Wildfire

Figure 130 - Selected by Type for the entire Center.

### Method 2 – Incident Type

#### Sorting by the incident type

As example,

- Selected Type radio button to retrieve all incidents by specific incident type.

- From dropdown select “Wildfire”

- Figure 131 shows the result of this sort.

If you select “Mine” instead of “Type” and “Wildfire” you will sort all incidents that are yours. You also do the same by selecting “Stage” and “Wildfire” to retrieve the incident status.

Figure 131 - Select using the dropdown menu for incident types.

Figure 132 – Sorted by Wildfire

### Export

All open incidents can be downloaded as a CVS file or printed.

- Select “Type” radio button to retrieve all open incidents.

- Select “Mine” radio button to retrieve your open incidents.

- Click “Export”

## Section 8: Daily Log Panel (F12 or Daily Log Icon)

Figure 133 - Daily Log Panel Icon

The Daily Log allows the user to enter log comments each day without the need to open a separate Incident just for this purpose. The user may also view all Incident and/or Resource activities for the day.

### Add Log Entry

- Select Category (if applicable) and change Initials if necessary.

- Type the ‘Log Entry”

- Click “Add” to save.

Figure 134 - Log Entry and Add

Figure 135 - Log Entry and Save

### Editing or Viewing the Daily Log by either Incidents or Resources or Both

- Incidents – shows information regarding the incident.

- Resources – shows information the dispatch location of a resource any time the status is changed. Changing a resource's status back to Avail Quarters, now shows its home location.

- Text/Email – posted to the Daily Log will include subject line and recipients names.

- Comments – shows from the “Comment Box.”

Figure 136 - Incidents on Daily Log Panel

Figure 137 - Resources on Daily Log Panel

### Highlight and Action

For both Highlight and Action complete the following:

- Select the log entry to Highlight (or Action)

- Check the box (Highlight or Action)

- Click “Save”

Figure 138 - Select entry to be Highlight or Actions

Checking “Highlight” shows the entry in red. “Action” shows in blue until the item’s status changes and action is turned off.

Figure 139 - Highlight on Daily Log Panel

# Part IV: Roster

## Section 1: Rostering

To be able to Roster a resource in WildCAD-E, a two-configuration step will be performed. The Center Admin will complete the first configuration step, which is to match the center’s WildCAD-E resources with the corresponding resources in IRWIN and IROC. Although we are matching the same physical resource, it is not necessary to name the WildCAD-E resource the exact same name as it has in IROC.

The second configuration step is to authorize persons to roster specific resources. An example might be that the same person located at a station might do the morning rostering for an engine and a dozer, and an FMO might have the right to roster those same two resources plus many others.

Dispatchers can only authorize those people who have the Roster role in WildCAD-E; so, if dispatchers are also going to do the actual rostering, they should not only have the Dispatcher role, but also the Roster role.

Figure 140 - Roster Menu

Clicking on Rostering, the menu will display resources that the user has been Authorized (Daily Routine Section). The Picklist Setup will be discussed in Section 2.

Left side of the Roster Panel (Figure 141) is the list of Resource that can be rostered by this user.

Figure 141 - Roster Panel, left side reflects the resources this user is authorized to roster.

The right side of the Roster Panel has two functions.

- The check box:

- if the box has a checkmark, that means that resource is available.

- If the box is blank, that means that the resource is out of service or on an assignment. The assignment status of Committed, Responding, On Scene, etc. will be shown.

- The Down Arrow:

- Click the down arrow up, and the staffing on the resource will be displayed.

Figure 142 – Rostered Resource and Adding Position

### Creating a Roster

Every Roster has a standard NWCG staffing pattern. As an example, the Engine, Type 3 has four positions. The user can add a fifth or more by clicking on the “Circle with Plus Sign.”

By selecting the “Circle with Plus Sign,” this action allows the user to select another position from the dropdown list.

Figure 143 – Selecting a Position

The next step is to select a person to staff the position.

There are two ways to select the person.

- From the Picklist – the personnel the users have assigned to their respective resource’s picklist will be visible and can be selected from the list.

- Name Search – in the line to the right of the desired position, start typing a minimum of three letters within the name. Once the name appears, click on it.

Figure 144 – Complete Rostering Resource

The rostered personnel are assumed to be qualified for the position they are filling.

- If the individual is not qualified a “Yellow Triangle” symbol will appear to the right of their name. WildCAD-E does not prevent the user from rostering a resource with a person who lacks the qualification. WildCAD-E will allow that assignment because at times there are delays in entering experience and training to maintain a person's qualifications. The bottom line is that managers and supervisors in the field have the final say over who goes out on what resources, not WildCAD.

- If the individual is a trainee, a “Green T” symbol will appear to the right of their name.

Figure 145 -Roster Status

Figure 146 - Red firefighter indicates resource is currently assigned.

- If the individual has a “Red Firefighter” with a line on it symbol, that indicates the person’s General Status in IRWIN is not currently Available.

- When adding an individual to a rostered resource, user will see individuals that are currently on another roster and will show the resource and dispatch center to which they are currently rostered. The resource is not selectable and appears grayed out until the resource is removed from the other roster.

Figure 147 – Example an individual currently on another roster.

## Section 2: Picklist Setup

Figure 148 - Picklist Setup is used by a Dispatch Center when a group of individuals routinely performs work together as a resource, such as an engine, etc.

When a dispatch center has a group of individuals who routinely perform work together as a resource, such as an engine, crew, etc., a Picklist can be created which makes dispatch more efficient. This step is optional, and users can roster a resource without creating a picklist. These lists are intended to save time.

When performing rostering in WildCAD-E, one can attach any person who has a record in IQS or IQCS to our engine crew, etc.

Figure 149 - Add a name to a Picklist.

To Set Up a Picklist

- Identify the individual resources that combine to create a resource appropriate for a Picklist (e.g., engines, crews, etc.).

- If an individual is added in error, click right of the name and once prompted, remove the individual resources from the list.

  - Click the up and Down arrow to view the names on the Picklist.

- Save your work.

Figure 150 - Example Picklist Setup

- When adding an individual to a Picklist, user will see individuals that are currently on another roster and will show the resource and dispatch center to which they are currently rostered. The resource is not selectable and appears grayed out until the resource is removed from the other roster.

Figure 151 – Example an individual to a Picklist that is currently on another roster.

Create a Roster

To create a roster for an Engine, assign an engine boss and crewmembers. A Picklist comes in handy for completing this task.

Figure 152 - Panel to create a roster.

- To create the resource, use the button in the upper left to add the position.

- To add additional members not on the picklist, type the individual’s name.

- A green “T” to the right of a name indicates a trainee status.

A user can add people only if:

- The current dispatch unit is their home unit,

- If the person is not presently on a filled request, and

- The individual is not presently on another roster in WildCAD-E.

If the user wants to assign that resource today, the user will need to contact the person who manages the roster for that other resource and have them release that person.

Once a resource is on an assignment, meaning they are on a filled request in IROC, substitutions or reassignments of any people are completed in IROC.

Those WildCAD-E users who have only the roster role come right to this screen when they log in, they do not have access to any other WildCAD-E features. The user clicks on rostering, and the screen will size itself well for use on a tablet, even a cell phone, so people in the field will have that option to do rostering from a computer or a tablet and potentially even a cell phone.

# Part V. Incidents

## Section 1: Search Incident Panel (F2 or Search Icon)

Figure 153 - Search Incident Panel

Each row in the search panel represents one method to search for an incident.

As example,

- Enter a date, from and to, and the information about what the user wants to search for on the incident (Figure 154).

- Click the Search button to execute the search. If the search was valid, the search will return a list of incidents (Figure 155).

- To clear an enter, Click the “Clear” button.

Figure 154 - Search Incidents using date only.

Figure 155 - Search results by date.

## Section 2: New Incident (F9 or Incident Icon)

Figure 156 - New Incident F9 Icon

### Create Incident

Incident can be created the following ways:

#### Manually

- Click the New Incident Icon without the location (Lat/Long)

- Manually enter the location on the Location (LOC) tab

#### Use the Drop Point

- Click on a location on the map, and to set a “Drop Point.”

- Click on the New Incident Icon on the lower right side of the map. This auto populates the Lat/Long and TRS on the Location (LOC) tab.

Figure 157 - Use the Drop Point to create an Incident.

Figure 158 – New Incident created by using the Drop Point.

#### The “Use Map” Button

- Click New Incident Icon or F9.

- Open the map and set the drop point location.

- Click the Use Map button and this auto populates the Lat/Long and TRS Location (LOC) tab.

Figure 159 - Use Map Location button to auto populate Lat/Lon.

#### Find Panel Icon

Click on the “Find Panel” Icon upper left of the Map and use any one of the methods for finding a location, then click the “GO” button.

Click on the New Incident Icon on the lower right side of the map. This auto populates the Lat/Long and TRS on the Location (LOC) tab.

#### Search Bar

- In the “Search Bar” on the Map, enter a place name, Lat/Long, etc., then click to go to a location.

- Click on the New Incident Icon on the lower right side of the map. This auto populates the Lat/Long and TRS on the Location (LOC) tab.

#### “Map” Button

If an incident has a location, the Map button in the incident header will be enabled. Click the Map button, the map panel will open (if it was closed) and zoom to a point located at the actual Lat/Lon of the incident.

Figure 160 - Map button in the incident header is enabled.

## Section 3: Incident Panel and Tabs

Creating an incident WildCAD-E, auto populates several items on the Incident Panel’s Header and LOC Tab.

### Header Information – Upper Portion

Figure 161 - Upper portion of Incident Panel Header

- Unit - Enter the Unit.

- Year – Enter the Current Year.

- Inc Num (Incident Number) – Is auto-populated by adding one number to the last used incident number. The last used incident number may be set up by the Center Administrator. The incident number cannot be changed.

- Type – For this Dispatch Center, the default type is “FI - Wildfire.”  Incident Types are set and cannot be changed. Appendix III – Incident Types contains the current list. Some Incident Types will have “Expired” dates, which will allow for searching, editing, and reporting incidents with expired types.

Figure 162 - Incident Type Dropdown Menu

Subtype

- Display “None,” which can be changed by using the dropdown menu.

- Incident Subtypes dropdown menu excludes expired subtypes from the list.

Figure 163 - Incident Sub-Type Dropdown menu.

- Status (as “Open”)

Before an incident can be set to close, the user must clear all resources assigned to the incident. The user will now get a prompt when attempting to close an incident that has resources assigned. The incident cannot be closed until all resources a cleared from the incident.

Figure 164 - Incident Status Dropdown menu.

The user will now get a Popup Confirmation when changing an incident status to Void.

Figure 165 - Void Incident Confirmation

- Incident Name - Displays “New,” which will be replaced automatically as the user type in the actual name. WildCAD-E will check for duplicate incident name in the same calendar year and indicate a duplicate with “DUP,” next the second incident name entered. Doing so will cause the sequence figure number to change. Incidents names with the “&” (ampersand character) in the incident name were not successfully to added to IRWIN.

Figure 166 - Incident Name

Figure 167 - Duplicate Incident Name

- Discovery Date and Time – Enter the date and time that the Incident was discovered.

#### Location of Tab Information

- Dispatcher – Name of Dispatcher at the time the incident was created.

- Response Area – Initially display “None,” which can be changed by using the dropdown.

Figure 168 - Completed Incident Panel Header

In WildCAD-E, two or three symbols are located on the right side of the ribbon panel. These symbols are:

- Lock – The Lock allows the user to lock or “Freeze or Un-freeze” the panel in place on the Home page.

- Three Dots – The three dots to the right of the lock, dock the panel on the Home page ribbon.

- The “X” – The “X,” to the right of the three dots on the Home page ribbon, closes the panel from view; but does not delete the panel.

Figure 169 – Symbols at the far right of the panel ribbon lock the panel in place; dock the panel on Home; and closes the panel from view.

Figure 170 – The Freeze Lock in the “Open” position allows the panel to move. The “Close” position docks the panel in place on the Home page.

Figure 171 - Example of a "Dock" Incident Panel.

#### Header Information – Lower Portion

On this portion of the Header, there are three buttons:

- Map Button – Zooms on the map to the incident location and displays a Drop Point.

- PDF Button - Prints this Incident Report to a PDF File.

- IWI Button – Creates an “Incident within an Incident” incident panel.

Figure 172 - Map Button

Figure 173 - Display the location of the incident after clicking the “Map Button.”

Figure 174 - The PDF Button

After clicking the ‘PDF’ button a pop-up will ask to confirm the inclusion of resource status.

Figure 175 - Report to include resource status or not.

Figure 176 - Example of a file created using the PDF button.

Figure 177 - Example of an "Incident Card."

Figure 178 - The “Incident with an Incident” (IWI) Button

Click on the “IWI” Button (Incident within an Incident)

Figure 179 - Adding a New IWI.

Then by clicking on the “Add New IWI” button that opens a panel has the same functionality as a normal incident panel and will associate this main incident with its own Tab on the original incident.

Figure 180 - Example of Adding a new IWI.

#### To add an existing IWI:

- Use the “Incidents” dropdown menu to select an IWI.

- Click the “Add Existing IWI” Button to add this existing IWI to the list.

#### To remove an IWI form the list:

- Click the box next to the “Incident ID.”

- Click the “Remove Selected IWI” Button to remove this IWI to the list.

- To remove more than one IWI at a time just click all the boxes next to the IWI you want to remove.

### IRWIN Integration

The fields required to have an Incident be sent to IRWIN are:

- “Disc Acres” - Discovery Size (new field “Disc Acres” on Location Tab)

- “Incident Name” - Once the user starts typing the word “New” will be replaced.

- “This center has primary responsibility” for this incident must be checked.

      - If the user’s center is NOT responsible for the incident, the user would un-check the box.

      - If the user UNCHECKS the box for a fire, the user’s information about this Incident will not flow through IRWIN to other systems.

Once the above is completed the incident panel will display “IRWIN Incident.”

Figure 181 - Example of IRWIN Incident.

#### Changing from an IRWIN Integrated Incident to Non-Integrated Incident

When changing from an IRWIN Integrated Incident to a Non-Integrated Incident type, any filled resource capability requests on the incident are closed and the IRWIN incident is set to invalid. Also, the following log entry is added to the IRWIN Log for the incident - “Changed to a non-integrated type, IsValid set to false.”

Changing the incident type back to an integratable type uses the prior IRWINID and sets IsValid=true (example, Wildfire accidentally changed to Misc, dispatcher notices the error and changes back to Wildfire).

When changing from a non-integrated incident to an IRWIN integrated incident type, the incident is sent to IRWIN once all the requirements are met. Filled capability requests are created for any IRWIN resources assigned (w/ status Responding, On Scene, or Returning) to the incident.

In the case where the incident type is changed to a "FI - Wildfire" the following rules are applied:

- When changing from an IRWIN integrated incident to a "FI - Wildfire" and the incident does not have discovery acres, the discovery acres are set to 0.01 by WildCAD-E. The incident continues to be integrated.

- When changing from a NON-integratable incident (i.e., Smoke Check, Misc), the discovery acres are now NEVER set by WildCAD-E

In the case where an escaped "FI - Wildfire" is created from a "FM - Prescribed Fire" (via the WF button on the incident header), WildCAD-E sets the wildfire's discovery acres to 0.01 regardless of the value in the prescribed fire's discovery acres.

The incident type cannot be updated for the following special cases:

- If the incident is part of an integrated parent/child relationship

"FI - Wildfire" related to a "FM - Emergency Stabilization".

"FI - Wildfire" related to a "FM - Fire Rehabilitation".

"FI - Wildfire" related to a "FM - Out of Area Response".

"FM - Prescribed Fire" related to a "FI - Wildfire".

- If the incident is a "FM - Preparedness/Preposition." When an incident type is changed to a "FM - Preparedness/Preposition,” the user is prompted with a dialog to confirm or cancel the change. The incident type can ONLY be updated to a "FM - Preparedness/Preposition" if the incident is not in IRWIN and the incident has no resources assigned.

#### Authoritative Data Source (ADS is Sit/209)

If the Center has turned over responsibility for updating the Incident to another entity (e.g., to an Incident Management Team), check “ADS is Sit/209” box. From then on, changes to the center data for this Incident will not flow to IRWIN. If the box was inadvertently unchecked and resources were then assigned, it was not possible to check it without releasing all the resources.

#### Incident Panel Tabs

As users work with WildCAD-E, they will frequently manage Incidents on this Incident Panel. The panel contains numerous "Tabs," each of which is described below. All Tabs have a set maximum length limit on all entry fields to avoid issues with user would try to save data with too many characters and the result would be that no data was saved.

### Location Tab (LOC)

Most of the information on the LOC tab is completed as the user creates the incident.

Such as,

- The “Current Location” is the dispatch location associated with the incident ‘s response area. I drop-down dispatch location is available to change the current location.

- For resources assigned to the incident, their current dispatch location will be set to the associated dispatch location for the incident.

- Added the ability to add half (x.5) decimal values for Township and Range.

The lower left side (Red Box) of the LOC tab has several free text areas that are optional. They can be viewed in Figure 182.

If the incident location is within an area that has been identified by the Center Admin as a “Hazard,” a warning “HAZARDS” will display below the free text area.

Scrolling to the bottom of the LOC tab will display the information about the hazard.

Figure 182 - Location Tab (LOC)

### Response Tab (RESP)

Manage the responding resources on this tab. RESP tab is NOT visible for incident types of FM - Preparedness/Preposition ("Prepo”) or FM - Out of Area Response:

Figure 183 - Response Tab (RESP)

The left side of the RESP Tab:

- Response Level - will default to the response level set in the Daily Routines. To start a response:

    - Use the dropdown to change the response level.

- Resource Status – will default to “Committed” once a resource is “Assigned.”

      - Use the dropdown to change the response status.

- Quantity - the preplanned number of resources to be dispatched at a select response level. These number are set in Center Admin within the “Dispatch Strategy,” The quantity are defined:

  - Re – Resource Type

  - Pl – Number preplanned

  - Or – Number requested (ordered) for this Incident.

  - Av – Number available

The user can change the quantities in the Ordered (Or) column to, for example, request the next closest engine.

#### Assigned Resources

The top portion right side of the RESP Tab includes:

- Assign -The Assign button commits the resource(s).

- Timer – Click on a resource, then the button will start a timer for that resource.

- Rem - The “Remove” button can be used to completely remove a resource from the incident, along with any associated actions on the Incident.

- Comment - When selecting a resource, you can enter a comment and change status to create a log entry.

#### Committed Resource

The middle portion on the right side of the RESP Tab includes:

- Select – Click on resource the use the dropdown “Resource Status” to change status.

- Clear - will clear the user’s last entry, such as select a resource and decided the user pick the wrong, click “Clear.”

- Type or Status – The user can sort the list of resources by Resource Type or Resource Status.

- Recommend – Click on the box to display the recommended response and click off the box and “Not Recommended” will be displayed. Current Dispatch Location is used for recommending resources.

#### Recommended Resources

The lower right side of the RESP Tab includes:

- Select – Click on resource and use the “Assign” button to commit the resource (multi select).

- Clear - will clear the user’s last operation, such as a select-a-resource and/or it is decided the user pick the error wrong, click “Clear.”

#### Selecting from the RESP Tab within the Recommended Resource portion:

To select ALL Resources:

- Click “Select” and each resource will have a red box around the resource.

- Click “Assign” which moves the resources to the Committed portion.

Figure 184 - Response Tab - Select

Within the Committed portion:

- Click “Select” and each resource will have a red box around the resource.

- Use the Resource Status dropdown to status the resources Committed, Responding, or At Scene, etc.

Figure 185 - Response Tab Assign

Figure 186 - Response Tab, Dropdown List

#### To Multi-Select SOME Resources

With in the Recommended Resource portion

- Hold the “Ctrl” key and click on the resources the user wants to commit (can be one or more than one resource).

- Click “Assign,” which moves the resources to the Committed portion.

Within the Committed portion:

- Hold the “Ctrl” key and click on the resources the user wants to commit (can be one or more than one resource).

- Use the Resource Status dropdown to status or to make the resources Committed, Responding, At Scene, etc.

Type or Status – the user can sort the list of resources by Resource Type or Resource Status.

Figure 187 - Resource Status - By Type

Figure 188 - Resource Status - By Status

Timer – Click on a resource, then the button will start a timer for that resource.

Figure 189 - Resource Status - Start Timer

#### Selecting from the Resource Status Panel

- Click on the resources the user wants to commit (can be one or more than one resource).

- Click “Assign,” which moves the resources to the Committed portion.

Within the Committed portion:

- Click on the resources the user wants to commit (can be one or more than one resource).

- Use the Resource Status dropdown to status or to make the resources Committed, Responding, At Scene, etc.

### Comments Tab (CMNT)

This tab shows information entered by the Center Admin for this response area.

Figure 190 - Comment Tab

### Actions Tab (ACTNS)

The Actions Tab shows all the status changes for resources on this Incident. When a resource is set to a status that makes it no longer assigned to the incident, the Actions tab now shows that the resource was "Released" along with the date/time. This record will also be in the status history for the incident.

Figure 191 - Actions Tab

### Distance (DIST)

These six lists are sorted according to air miles, with closest shown at the top. Each time the user visits this tab, the lists are recalculated based on the Lat/Lon shown on the Location tab. Expanded the geographic declination lookup used by WildCAD-E to include areas outside the continental United States so that the bearing calculations could be performed in Alaska and other areas outside the U.S.

Figure 192 - Distance.

### Frequency Tab (FREQ)

#### Adding a Frequency

- Under “Frequencies Type,” use the dropdown to select a category (e.g., Ground).

- Once a category is selected, a list of “Default Frequencies” descriptions will be displayed.

- Click the appropriate box adjacent to the frequency type to be used.

- Click the “Add” button to add a frequency type, and a description will be added to the list.

Figure 193 - Adding a Frequency -Type

Figure 194 - Example of Completed FREQ Tab

#### Deleting a Frequency

- Click the box next to the frequencies type that will be deleted. The “Trash Can” icon will be highlighted.

- Click the trash can icon. and a warning will appear asking to either accept “OK” or “Cancel.”

- Click OK to delete or cancel to make no changes.

- Click “Save” for either case.

### Incident Log (LOG)

#### Adding an Entry into LOG Tab

- Click on “Log Entry,” and type the entry.

- In the “From/To,” enter the name or initial for the individual the message is coming and going to.

- Click “Enter” key or “Add” button to save the entry.

- The Dispatcher’s initials will be displayed under the “User” column.

#### Editing a LOG entry

- Click on the entry line the user wants to edit and the entry will be displayed in Log Entry line.

- Make the edits.

- Click the “Save” button.

- Click “Save” for either case.

#### Voiding a LOG Entry

- Click on the entry line the user wants to void, and the entry will be displayed in Log Entry line.

- Click the check box next to “Strikethrough” to indicate this entry was void.

- Click the “Save” button.

#### Text/Email

Clicking on the Text/Email button takes the user to the Text/Email panel on a separate tab. Text messages sent from within an incident will now appear in the Incident Log and the Daily Log

Resource Status Changes

Resource Status Changes only appear on the Log if a “Comment” has been included with the status change.

#### Timer

Click on the Timer Icon to start a timer.

Figure 195 – An example of LOG Tab.

Figure 196 -An Example of void LOG entry (Strikethrough)

### Number (NUMBR)

The Center Admin creates categories of numbers (Descriptions), which the users can then enter the “Value.”

- Text or Non-changeable Number are entered directly.

- Sequence of Number is set up as an “Use Auto” by the Center Admin, the user will click the “Next Number” button to retrieve the next number. This will change the number to +1 of the last number that was on the Numbers Tab.

Figure 197 – Number – Add Numbers and/or Text.

### Notifications (NOTIFY)

The Center Admin creates the Notifications list. Once the list is available:

- Click on the line of the entity to be notified.

- Under “Comment “column, enter brief details about the notification and click to text notification.

- Click “Save,” and the date and time will be automatically populated.

If certain Notifications are required for this Incident, they will appear at the top in red.

Figure 198 - Notify Tab.

### Contracts (CONT)

By selecting, the “Type” or “Sub Type” of contract, a list of known contractors will be displayed and sorted by air miles to the Incident.

- To see information about a specific EERA/Contract, simply click on that respective contractor from the list.

At the bottom of the panel, enter the “Date” and “Resource Order #,” and select appropriate status for the Contractor on this order:

- F – Filled

- D – Declined

- U – Unable to Contact

Figure 199 – Selecting a Contract

Click “Add” to add this history for the Contractor.

Figure 200 – Add the Contractor and appropriate status.

### Fires (FIRES)

Figure 201 – The FIRES Tab

The FIRES Tab is used for two different purposes:

- On the left, enter the “Initial Report of Conditions” using free text along with a dropdown menu for “Initial Strategy,” enter the following:

- Fuel

- Jurisdiction

- Acres

- Wind Speed

- Direction

- Slope

- Initial Strategy

- Access

- Hazard Concerns

- On the right, enter the final “Fire Report Information,” which will be used for the fire summary reports.

The “Auto” buttons will automatically assign the next Fire Number and next Sub-Unit Fire Number.

- “Acres” and “Cls” (Fire Class - A, B, C, etc.)

- When the user changes acres, the class will automatically be updated.

#### The Contain, Control, and Out Date

- Enter the date and time for each and use the “Clear” button to change entries:

    - The Contain (date and time) can’t be before the Discovery Date and Time but can be equal.

    - The Control (date and time) can’t be before the Contain Date and Time but can be equal.

    - The Out Date (date and time) can’t be before the Contain Date and Time but can be equal.

If any Date/Time are before each other, an alert will be displayed in a red line and highlighting Date in Red along with a red line.

Clear button for Contain/Control/Out dates to allow clearing the value. Clearing Contain/Control/Out dates clear their corresponding values in IRWIN.

Once the user enters the Out Date (date and time) WildCAD-E will lock several entries while other Systems are finalizing the Incident.

“Fire Cause” – Use the dropdown to set the cause as one of the three classifications below:

- Undetermined (Default)

- Human

- Natural

On the lower portion of the of the FIRES Tab, the user can enter additional information, such as:

- Initial Report Information

- Fire Report Information

Figure 202 - An Example Initial Report Information

Tip:  Press CTRL + Enter to post.

Figure 203 - An Example of Fire Report Information

The “Fire Growth History” is a summary of acres from the entry above.

Figure 204 - Fire Growth History is a summary of the acres.

### Incident Command Post (ICP)

Enter information about the Incident Command Post on this tab.

#### ICP Location – Free Text

The user can select the following by clicking one of the check boxes:

- “Use Incident Lat/Lon for ICP”

- OR enter specific Lat/Lon for ICP; and then, type in the Lat/Lon.

Enter with free text:

- Description

- Staging, Helispots or other specific information.

The Functions in the lower portion of ICP Tab are pre-established, and all the user fills in is the respective phone numbers.

Figure 205 - Incident Command Post Tab.

### Aircraft (AC)

The following portions of the AC Tab are automatically populated:

- Legal

- Frequencies

- ATB, Helibase, VOR, Hospital, and Burn Center

Enter free text for:

- Flight Restrictions

- MTR/SUA

- Hazards

- Other Aircraft

- Reload Bases

- Ground Contact

Figure 206 – An Example of the AC Tab

### Move Up (MU)

On the left side of the tab are the “Pre-planned Move Ups” for this Response Area, Response Type, and Level are shown.

Select the resource to move up.

- Click the “Move Up” button.

- Then, on the right side of the tab select:

  - Resource to move from the dropdown (e.g., E337LPF)

  - Location for the resource to move to (e.g., Apache)

  - Click the “Move UP” for the additional move up.

Figure 207 - Move Up Resources

### Incident Commander (IC)

Use this tab to create a history of the Incident Commander. The user may also enter Trainees. To do so, enter the same information, and click in the box below “Trainee” and type “yes.” The grid sorts by Created Date in descending order.

Figure 208 - An Example of the IC Tab

(TIP: Press Ctrl + Enter to post.)

### Questions (FI - Wildfire or varies)

The name of the Tab between IC and Fiscal will change according to the Incident Type and will contain additional questions (for the selected Incident Type). This information is developed for use by the Center Admin.

To enter information for a question, click on the question, and type the response in the “Answer” area below. Then, click “Save.”

Figure 209 - Example of a Question Tab for Wildfire

### Fiscal (FISC)

Once an Incident has gone to IRWIN, the user can have WildCAD-E automatically request a FireCode via IRWIN. This is done on the Fiscal Tab by pressing the “Get FireCode” button.

This button is only enabled for use after all the required fields have been successfully sent to IRWIN, and IRWIN has accepted them.

After clicking “Get FireCode,” wait several seconds and the Auto FireCode will appear. The user cannot edit this Auto FireCode.

The user may manually enter additional fiscal codes.

These codes will be sent to IRWIN. They include:

- ABCD Misc.

- U.S. Forest Service (FS) Job Code

- FS Override

- State Fiscal Code

Clearing ABCD Misc, FS Job Code, and FS Override clear their corresponding values in IRWIN.

However, Other Fiscal Codes will not be sent to IRWIN. The user may manually check the box for inclusion within the financial report. The following may be checked.

- FS Assisted

- Multi-Jurisdictional

- Trespass

- Reimbursable

Use the dropdown menu to select “Unprotected Response Reason.”

- Threat to Protected Lands

- Burned Onto Protected Lands

Figure 210 - Example of a FISC tab.

### IRWIN Status (IRWIN)

This tab displays information about the Incident and IRWIN:

- The Date/Time the IRWINID was received (when IRWIN accepted the Incident).

- When a FireCode was requested.

- When a FireCode was received.

- Any error messages from IRWIN in response to the user’s attempt to update this Incident in IRWIN.

- To show all updates check the box next to “Show all updates.”

Figure 211 - An Example of an IRWIN Tab.

TIP: Ctrl + Enter = Post

### Conflict Status (CONF)

Prior to submitting a new Incident to IRWIN, WildCAD-E checks for any Incident conflicts. A conflict is defined as another Incident already existing in IRWIN which:

- Is managed by a different Dispatch Center (Dispatch Center ID)

- Is within ½ mile (Initial Latitude and Initial Longitude)

- Was discovered within 6 hours (Fire Discovery Date and Time)

Figure 212 - Conflict Status - No Conflict

If IRWIN detects one or more potential conflicts, WildCAD-E will display the background in RED alerting users to the fact that the incident conflicts with another IRWIN incident. CONF tab is only visible for incident type “FI - Wildfire.”

Figure 213 – Conflict.

If a new Incident has potential conflict(s), the Dispatch Center managing the Incident has the primary responsibility for resolving the conflict(s).

Figure 214 –– Conflict Resolved

Select one of the following solutions:

- My incident is a legitimate fire managed by this center.

- This means there are “no duplicates.”

Release all resources to their home location prior to selecting the next 2 options.

- My incident is NOT a valid incident and will be VOIDED.

- This means to change this incident to “Void.”

- My incident is a duplicate of the one above and is managed by the other center.

- This means that the resulting conflict resolution will now be set to incident type of "FM- Out of Area Response."

In some cases, the other system did not clear the quarantine. Those conflicts are now left on the CONF tab for the dispatcher to clear.

### Incident within Incident Status (IWI)

An IWI is created on the LOC Tab as discussed earlier. The IWI Tab lists the IWI information, which includes:

- Incident ID

- Name

- Type

- Date and Time

The user can also add additional IWI from this IWI Tab, including:

- “Add New IWI”

- The “New” IWI will be automatically added to the list.

- A new Incident Panel will open, and the user can begin to enter in the information regarding the IWI.

The user can also, remove an IWI:

- Click on the check box for the IWI.

- Click the “Remove Selected IWI.”

The user can also, add an IWI to a different existing Incident or IWI:

Use the Incident dropdown to select an existing incident.

- Click the “Add Existing IWI.”

- The “New” IWI will be automatically added to the list.

WildCAD Incident Card (PDF) - The report header will contain "This incident has IWIs" followed by a list of children incidents. Note: Each child incident is generated as a separate PDF when the report is run for the parent incident.

## Section 4: Escape Prescribed Fire. Post Fire Events and Out of Area Response

Section 4 includes the following topics:

- Prescribed Fire Escapes (FI – Prescribed Fire)

- Post Fire BAER (FM – Emergency Stabilization)

- Post Fire Rehab (FM – Fire Rehabilitation)

- Fire Support (FM – Out of Area Response)

For both Prescribed Fire and Post Fire Events, a red “REL” tab will be displayed on both the original Prescribed Fire or Wildfire and the Escape Prescribed Wildfire and Post Fire Events. These incidents must be related to IRWIN incidents. The related incidents are listed and can be opened from the list by double-clicking.

### Prescribed Fire Escapes (FI – Prescribed Fire)

To create a Prescribed Fire incident panel:

- Step 1 - Create the “Incident Panel” like a Wildfire.

- Step 2 - Select the Incident Type “FI Prescribed Fire”

- Step 3 – Name to Incident with “RX” within the name

Note – No Discovery Acres are required for the Prescribed Fire to become an IRWIN incident but can be entered.

Figure 215 - Prescribed Fire Incident Panel

To create an Escape Prescribed Fire a “WF” button is now on the header. The user will create a new wildfire incident that is related to the prescribed fire by clicking the “WF” button. If there are resources on the Prescribed Fire when it escapes, the resources will not be reassigned to the Wildfire. The user can reassign the resources at any time.

Figure 216 - Escaped Prescribed Fire Button

Once you click on the “WF” button a pop-up will require the user to confirm “Create Escape Wildfire”

Figure 217 - Create the Escaped Prescribed Fire

In IRWIN, a "Prescribed Escape" relationship is created between the two incidents with the RX as the parent and the WF as the child.

- The wildfire discovery date/time is defaulted to the current date/time.

- The wildfire incident name is the same as the prescribed fire with the “RX” removed from the name.

- All the incident header and “LOC” tab data elements from the Prescribed Fire are copied to the related Wildfire.

- The Discovery Acres on the Escaped Wildfire area set to 0.1 acres if the Prescribed Fire does not have a value for discovery acres.

- Once the two incidents are related, the incident type for both the parent Prescribed Fire and the Wildfire cannot be changed.

Figure 218 - New Wildfire Incident Panel

Note the Incident Type and Name.

Click on the “REL” tab to view the related Prescribed Fire and the user can also click on the Incident listed to open that incident.

Figure 219 - Related Prescribed Fire Information

The Prescribed Fire Incident will have the same “REL” tab to view the relationship between Prescribed Fire and the Escape Wildfire.

Figure 220 - Related Prescribed Fire Information

### Post Fire BAER (FM – Emergency Stabilization) and Post Fire Rehab (FM – Fire Rehabilitation)

For incident type "FI – Wildfire," two new buttons are visible for creating related post wildfire incidents - "Post Fire BAER" and "Post Fire Rehab."

Figure 221 - "Post Fire BAER" and Post Fire Rehab" Buttons.

Both the "Post Fire BAER" and "Post Fire Rehab" incidents are created in the same way. Once user clicks on the either "Post Fire BAER" or "Post Fire Rehab" button a pop-up will require the user to confirm the “Create Post Fire.”

Figure 222 - Create the Post Fire Incident

In IRWIN, a "Post Fire" relationship is created between the two incidents with the Wildfire as the parent and the BAER or Rehab as the child.

- The BAER or Rehab discovery date/time is defaulted to the current date/time.

- The BAER or Rehab name is the same as the Wildfire with the “BAER” or “Rehab” added to the front of the incident name.

- All the incident header and “LOC” tab data elements from the Wildfire are copied to the related BAER or Rehab.

- Once the two incidents are related, the incident type for both the parent Wildfire and the BAER or Rehab cannot be changed.

Figure 223 - New BAER or Rehab Incident Panel

Note the Incident Type and Name.

Click on the “REL” tab to view the related Wildfire and the user can also click on the Incident listed to open that incident.

Figure 224 - Related Prescribed Fire Information

The Wildfire Incident will have the same “REL” tab to view the relationship between BAER or Rehab.

### Fire Support (FM – Out of Area Response)

For IRWIN incidents type "FI - Wildfire" with NO resources assigned, the user can now change the type to "FM - Out of Area Response," when the incident is NOT part of an IRWIN conflict.

If resources are still assigned the checkbox will change to said “This center has primary responsibility (release resources first)” when there are resources assigned to the incident

Figure 225 – Resources still assigned to the incident

In this example, FI – Wildfire (A-053) is the supporting (OR) incident and FI – Wildfire (A-075) the parent incident.

Figure 226 - Incident A-053 (OR)

Figure 227 - Incident A-075 (Parent)

To create a "FM - Out of Area Response:"

- Step 1: Uncheck the "This center has primary responsibility," checkbox.

- Step 2: Enter the “Incident being Supported” Unique Fire ID (e.g., 2024-CABDF-240075) of Supported Incident.

- Step 3: Click the "Get Inc Info" button.

Figure 228 - Creating an FM - Out of Area Response

Once you click on the “Get Inc Info” button, a pop-up will require the user to confirm “Create Relationship.”

Figure 229 - Create Relationship

YES, will change the incident type to an OR (FM - Out of Area Response) and create an IRWIN "Providing Response To" relationship to the parent incident (A-075). For this type of OR there is no “REL” tab, the providing support incident Fire ID is to the right of the “IWI” button.

Figure 230 - Supporting Incident A-053.

## Section 5: Timers

### Open Timers (F4 or Open Timer Icon)

Figure 231 – Open Timer Panel Icon

Figure 232 - Open Timer Panel

- Enter a date under “Show Timers After,” to show a timer after a certain date.

- Click the “Search” button.

Figure 233 - Show Open Timers

Figure 234 - Show Closed Timers

By default, only open timers are displayed. To show Closed Timers:

- Click the “Show Closed” timer box.

- Click on a timer to open in that timer panel.

Figure 235 - From the Panel, Open a Timer.

### Create New Timer

Figure 236 - New Timer Panel Icon

WildCAD-E allows the creation of “Timers,” which remind the user to act after a certain number of minutes.

To start a New Timer:

- The user will select the Timer Icon or use F4 to see the Timer screen (shown in Figure 236).

Figure 237 - New Timer

To Select the Resource to be monitored:

- Use the dropdown or enter the resource manually.

The following entries are automatically populated:

- Time From– The time the alert will start and will change after the “Status OK’ button is pushed.

- Interval – The number of minutes before the timer wants an “Ok Status.” This time is set by Center Admin.

- Alert Time – Number of minutes until the alert will end and will change after the “Status OK’ button is pushed.

- Rem (Remaining Time) – The time counts down from the set start minutes and will restart after the “Status OK” button is pushed. T

Click the 'Enter' key, after typing text, this functions the same as clicking on "Status OK" or "Other" buttons.

Aircraft have additional optional fields to complete if the user selects “Aircraft” as their resource:

- Lat/Lon

- Heading

- Altitude

- Speed

- Passenger

- Fuel

Figure 238 – Select Resource Timer

Figure 239 – An Example of a Typical Timer

### Sound

Check the box next to “Sound,” and sound will occur automatically,

- Uncheck the box to disable the sound.

Alerts – Two different alerts are visible.

- The first alert occurs when the time remaining (REM) reaches zero minutes. This alert will be a “Flashing Red Line” around the timer. The timer countdown continues after zero showing negative count instead of stopping at zero.

- The second alert occurs if no action is taken by minus (-) minute. This alert will be a “Flashing Red” fill the entire timer.

Figure 240 – Flashing Red Line Around Resource Timer

Figure 241 - Flashing Red fills the entire Resource Timer

The user can “Dock” a Timer, just like with the Incident Panels, by using the three dots in the upper right corner of the Timer panel.

- If docked the same automatic alerts will occur, will become undocked, and cannot be docked again.

- The timer will remain on top of all other panels.

Figure 242 - Docking AA07LPF 6 Resource Timer.

Figure 243 - Alert for Docking AA07LPF 6 Resource Timer

Figure 244 – Snooze button timer.

These buttons (in Figure 244) control the management of the timer:

- Mute – Use the button to stop the Alert Sound.

- Close – Use the button to end the Timer.

- Snooze - Use the button to put the Timer on hold.

After a timer has been status in “Snooze,” the button will automatically change to “Resume.”

- To re-start the timer, click on the Resume button (Figure 245).

Figure 245 - Resume Timer

## Section 6: Complexes

Figure 246 - Complex Icon

### Create a Complex

When the “Complex Icon” is initially clicked (Figure 246), the user will be prompted to confirm a complex is intended to be created before the Complex can be created.

Figure 247 - Dispatcher will confirm creation of a Complex incident before the action is taken.

- “IRWIN Created” will not display until the Complex name is entered.

Figure 248 - Creating a Complex

After clicking on the Icon, WildCAD-E auto populates the following list of items on the Complex Panel’s Header:

- Unit - Uses the default as set by Center Admin.

- Year – Uses the current calendar year.

- Inc Num (Incident Number) – The Center Admin may have added a prepend with calendar year and/or an ending suffix to the end of the incident number. This is common when a Center defined code is used to identify different units within the same Center.

- Discovery Date and Time – Date and time incident was discovered.

- Status (as “Open”)

- Lat/Lon – The initial Lat/Lon is a temporary location until the Incident is added to the Complex. Once one incident is added the Lat/Lon becomes the location. As more incidents are added the Lat/Lon is calculated to a center point of all incidents.

### Incident Name

- Displays “New Complex” until the user types in the actual name of the Complex.

- “New Complex” is automatically replaced at this point.

- For every Complex, the name MUST include the word “Complex,” as example “A-562 Complex.”

### Log and IRWIN Buttons

#### Log Button

The “Log” button opens a screen like the Incident Panel Log Tab.

#### IRWIN Button

The “IRWIN” button displays:

- IRWIN ID

- ADS Permission State

- History of transaction with IRWIN

### Adding an Incident to the Complex

- Select an incident from the Open Incident Panel.

- Click “Assign” at the bottom of the Complex panel. (An Incident must have an IRWINID to join a Complex and be a Wildfire.)

- Click “OK” to confirm.

  - The “OK” action will automatically change the Lat/Lon to the add incident Lat/Lon.

Figure 249 – Add an Incident to a Complex

2

1

Figure 250 - Confirmation that the user wants to add an Incident to a Complex.

3

Figure 251 - The Added Incident to the Complex

There is no limit to the number of Incidents (wildfires) that the user can attach to an Incident Complex. The user can add or remove Incidents from a Complex at any time based on incident management strategies (see the section Release an Incident from a Complex).

### FireCode

- After adding an incident, click on the “Get Firecode” button, and the FireCode number will be displayed.

- A FS Job Code and FS Override can be entered using free text.

Figure 252 - Get Firecode Button Panel

Figure 253 - The FireCode is displayed.

### Releasing an Incident from the Complex

- Click on the Incident to be released.

- Click the “Release” button.

Figure 254 - Confirmation that an incident is to be released from a Complex.

## Section 7: Field Interrogation File (F3 or FI File Icon)

Figure 255 – FI Icon

### Field Interrogation File (FI File) panel

Law enforcement incidents may be used by any dispatch center. If a center wants to use the FI system, they need to request access with a Service Request to Bighorn Information Systems.

Figure 256 - Unless the dispatcher is authorized and "LE Authorized" in the system is set to "yes," they will not have access to navigate to and use the FI File panel.

The Center Administrator will grant a Dispatcher “LE Authorized.”  Only dispatchers who are “LE Authorized” can create, edit, or view law enforcement incidents or FI records.

Figure 257 - Dispatcher set to 'yes' is authorized for the FI System.

The “Open FI File (F3)” button on the Home Page ribbon opens the FI screen for search or creating an FI record does not tie to Incident. The Home Page “FI” Icon does not allow starting a LE Incident. From the Home Page only one instance of the FI File search panel can be open

Multiple FI File panels that are associated with incidents can now be opened simultaneously multiple Panel by clicking incident panel.

To search the FI record by:

  - Resource

  - Vehicle Plate

  - Name and or DOB

  - Contact Date and Time (Saves automatically by clicking “Save New” or “Save Edit” Button)

  - Op Lic State and or Number

  - Firearm

  - View a description (Save by clicking “Save New” or “Save Edit” Button)

  - Open a record associated with the search.

Figure 258 - FI Panel

### Searching by Resource (Block 1)

The only resources that will show in the dropdown will be Resources that the Center Administrator indicated that the Resource is authorized the use of the law enforcement Field Interrogation File.

- Either type in a Resource Name (Code)

- Click the “Show All” to the list of resources authorized.

- Then click the “Search” Button near the bottom of the FI Panel

Figure 259 - Search by Resource

This will return any records associated with that resource.

- The record will show in Block 8

- Click on any record associated with the search.

- As example this search only had one record,

- Add any changes and click the “Save Edit” button.

Figure 260 - Results of Search by Resource 10EDW.

### Searching by Name/DOB or Op License State/Number or Firearm

In each of the Blocks 2, 3, 5 and 6 enter a minimum of one search criteria.

As example,

- In Block 2 the search criteria used was last name only (Booher)

- Enter “Booher” and click the “Search” Button in this block.

- This search criteria displayed four (4) records for the last name. (Figure 261)

- Single click on one of the records will fill in the data regarding that record.

- Double click will open the incident (B3-1031) associated with that record. (Figure 262)

Figure 261 - Results of Search by Name

Figure 262 – Result of Search Incident

# Part VI: Phone Directory

## Section 1: Using the Phone Directory

Figure 263 - Phone Directory Panel

The Phone Directory will open in its own Tab.

Figure 264- Phone Director Dropdown

### To Refresh the Phone Directory

- Use the “Refresh Icon” to refresh the entire phone directory.

### To Look Up a Person in the Phone Directory

- Select from the dropdown and select Search by “Last Name.”

- Type in person’s last name.

- Click the “Search Icon.”

  - Information for that person or persons with the same last name will be displayed.

  - The user will select the correct individual.

### To Clear the Search

- Use the “Clear Icon” to clear the search.

### Adding a person to the Phone Directory

- Click the Plus (‘+”) Icon and a new line will appear.

- Enter the required information under each column.

- Click the “Save Icon.”

# Part VII: Text/Email

## Section 1: Using Text/Email

Text/Email opens in its own Tab. The Center Admin has several roles, responsibilities, and tasks to complete within the Text/Email function that must be completed before the Dispatchers can use this function. Dispatch Center Members is one area that must be completed by the Center Admin

Figure 265 - Dispatch Center Members

### Two Ways to Send Text/Email Messages

- Select Names of Recipients – usually more than one is selected.

- Group Name – This is a pre-loaded list of recipients (created by the Center Admin).

### Select Recipient Name Text/Email Message

The users can:

- Select who will receive the message by clicking on the check box next to the respective name.

- Click on the “Add to Recipient List.”

  - The name(s) and Text/Email addresses will be displayed on the right side of the panel.

- IF the message is regarding an Incident, click on “Re” to select an incident; and then, click on the “Add Map Link” for that respective incident.

- Enter the “Subject” using free text.

- The "Subject" is included in the text after any "Always Start With" and before the body.

Figure 266 – Always Start Your Message with . . .

- When a Text is sent related to an incident, the subject now includes the incident information formatted as Year - Protecting Unit - Incident Number and Incident Name. For example: "2023-CABDF-000023 Bald Mountain" plus text of subject if added by user.

Figure 267 – Text related to Incident.

- On “Message” line enter the message using free text.

- Click “Send” and send the message to the people on the Recipient List.

- Text messages sent will now appear in that user's Daily Log/Incident Log.

Figure 268 - Select Dispatch Center Members

Figure 269 - Add to recipient list.

Figure 270 - Recipient List

### Compose Preset Messages

- Select the previously composed message from the “Use” dropdown.

- Enter the required information for the item in the preset message.

- The subject for pre-set messages is now autofill by defaulting to the canned message name.

- Follow all the steps to either send as a “Name Select” or “Group Select” message.

Figure 271 - Sent Message

### Selecting a Group Name Text/Email Message

- Select the “Group Name” from the Dropdown.

- Click on the “Add to Recipient List.”

  - The names and Text/Email Addresses will be displayed on the right side of the panel.

- If the message is regarding Incident, click on “Re” to select an incident and then click on the “Add Map Link” of that incident.

- Enter the “Subject” using free text.

- On “Message” line enter the message using free text.

- Click “Send” to send the message to the people on the Recipient List.

Figure 272 - Group Select.

Figure 273 - Example Text Message

# Part VIII: Reports

## Section 1: Available Reports

Reports Tab

Reports will open into its own Tab.

Figure 274 - Reports Menu Screen

The list of reports stated above will be increased over time. Below are examples of the main reports.

- Summary Graphs/Report

- Normal Response Run Card

- Daily Log Report

- Morning Report

- In Service Report

- Timer Reports

- Custom Incident Report

The remaining reports for Resources and Incidents are in Appendix II.

### Summary Graphs/Reports

The Summary Graph/Report can be created by selecting:

- Date Range – From and Through.

- Type – From the dropdown menu select specific incident type or none for all incident types.\

- Unit – From the dropdown menu select a specific unit or none for all units.

- Radio Button (Type, Day, Hour) – Select the radio button for the summary report and graph.

- Last step – Click on Pie or Bar to create the summary.

- Clear – to start over and clear current summary.

- Print – To print the summary to a PDF. After clicking on the Box next to Resources, from the dropdown menu select an individual resource.

Figure 275 - Summary Graph Reports by Type

### Normal Response Run Card

The Normal Response Run Card can be created:

- Singular – Click on a response area.

- Select All – Press Ctrl + A inside of the Response Area Box

- Multiselect – Holding down Ctrl while selecting the response areas.

- Clearing Multiselect – Click on any response area without holding Ctrl.

The following optional sections can be included in the report output by clicking on the boxes next to these sections.

- Prioritized Dispatch Locations

- Resource Quantities by Type

- Comments

For any one of selection criteria, next select the “Response Type” from the dropdown menu.

Figure 276 - Creating a Normal Response Run Card without boxes checked.

At this point, the user would click “Search” to view the Normal Response for a response area.  The panel will display a spinner and snackbar alert for when the data is loading.

Figure 277 – The data is being loaded

If the data being loaded is too big a detailed messages that the report data has failed.

Figure 278 – The Report has failed

Figure 279 - Searching a Normal Response Run Card

Click on the boxes next to:

- Prioritized Dispatch Locations

- Resource Quantities by Type

- Comments

This will your selection to be added to the Normal Response for a response area.

Figure 280 - Creating a Normal Response Run Card WITH the boxes checked.

Also, from this panel you can create a “PDF” for this response area. If you selected all or multiselect the search and PDF would be generated to all selected.

Figure 281 - WildCAD-E Run Card

### Daily Log Report

The Daily Log Report can be created by selecting:

For daily entries:

- Date Range – From and Through the daily log entries will be displayed.

- Category (Optional) – that was created by Center Administrator

For Incident and Resource entries:

- Incident and or Resources - Include one other or both.

- Resources – After clicking on the Box next to Resources, from the dropdown menu select an individual resource.

Figure 282 - Creating a Daily Log Report

Once you have created the version of the report click “Generate PDF” to save.

Figure 283 - Sample Daily Log Report

Figure 284 - Sample Daily Log Report for Incidents

Figure 285 - Sample Daily Log Report for Resources

Figure 286 - Sample Daily Log Report for both Incidents and Resources

### Morning Lineup Reports

Figure 287 - Morning Line Up Report

Figure 288 - Example Morning Line Up Report

### In Service Report

Figure 289 - Example of In-Service Report

Figure 290 - PDF Morning Line Up Report

### Timer Report

Figure 291 - Timer Report

Figure 292 - By Incidents and Non-Daily Log Report

### Custom Incident Report

This report allows the users the ability to create custom reports for incident data.

- Select date range.

- Incident types and incident subtypes (if any). The default incident types are Wildfire, Vehicle Fire and Structure Fire.

- Select columns to be displayed.

Figure 293 - Custom Report Description

Figure 294 – Example of Steps 1 – 3.

At this point you can generate a PDF report by clicking on the “Generate PDF’ button.

Figure 295 - PDF file is created,

#### Example of the Report

Figure 296 - Example of the Report.

#### To save a report

- Give the report a “Report Name.”

- Click on the “Save Report’ button.

Figure 297 - To save the report.

#### To Retrieve a Report

- Use the dropdown menu under “Incident Reports.”

- Click the named report that has been saved.

- Existing reports cannot be edited. Create a new report if changes are needed.

#### To Delete a Report

- Select an existing report.

- Click the “Delete Report’ button.

Figure 298 – Save a Report.

# Part IX:  Links

## Section 1: Websites and Documents

### Websites

The Links Menu will only appear if the Center Admin created a list of web pages for use by Dispatchers. If the menu is shown, merely select any one of the listed sites.

Right-click on the display to pop up a menu that allows the user to perform tasks such as going Back, Forward, Printing, etc.

### Documents

If the Center Admin established a list of documents in WildCAD-E, the user can open one or more documents from this menu. Examples might include User Guides or word processing templates.

The Links menu expands when the user clicks on the “>” symbol.

A list of Links Categories will be displayed.

Figure 299 - Links Menu Categories

The Links Categories (“AGENCY’) expands when the user clicks on the “>” symbol.

A list of Links Web Sites or Documents will be displayed.

Figure 300 - List of Web Site or Documents

Select for example “WildCAD-E Training,”

- Upon selecting “WildCAD-E Training from the list, a new tab will open with the website loaded.

- If the user goes back and selects another website, the “WildCAD-E Training” website would close and be replaced with the next website.

  - Only one web site is open at a time.

Figure 301 - Website Link

# Part X: Appendices

## Appendix I – Icons and Function Keys

When a function key is available, users can either use the icon or function keys to access those panels within WildCAD-E.

Table 1- Table of WildCAD-E Icons

| WildCAD-E Icon | Function Key | WildCAD-E Icon Definition |
| --- | --- | --- |
|  | F9 | Create a New Incident Panel Icon - Starts a new Incident, although the user will have to enter all information such as location and response area. |
|  | N/A | Create a New Complex Panel Icon – There is no function key available for “Create a New Complex Panel.” Users will use the icon to access this function. |
|  | F8 | Open Incidents Panel Icon – Opens the screen that displays existing, open Incidents. |
|  | F2 | Open Incidents Panel Icon – Opens the “Search Incident Panel” for the user. |
|  | F7 | Resource Status Icon – Opens the screen where the user can perform status changes that are not related to Incidents, such as “In Service.” |
|  | F12 | Daily Log Panel Icon – Opens the screen that display the “Daily Log,” where the user can document the activities not directly related to a specific Incident. |
|  | F5 | Map Panel Icon – Opens the WildCAD-E map. |
|  | F4 | Open Timers Panel Icon – Opens the Timers Panel for the user. |
|  | N/A | Create a New Timer Icon – There is no function key associated with the “Create a New Timer” icon. Users will use the icon to access this function. |
|  | F3 | Create a New Field Interrogation Fire (FI) or be able to search existing FI files. |

## Appendix II - Grids

Grids are a spreadsheet style interface with a variety of features available to those with Center Admin access to WildCAD-E.

### Sort Order of a Grid

- Click on any column heading or label.

- Sort by ascending (alphanumeric) by clicking once on the arrow pointing up.

- Sort by descending by clicking once on the arrow pointing down.

- For no sort, click a third time.

- When adding a new record or editing an existing record if sorting is enabled, it can make the record jump to a different location due to the sorting.

Figure 302- Arrow pointing up indicates a sort by ascending.

Figure 303 -Arrow pointing down indicates a sort by descending.

Figure 304 - Click three times to choose no sort.

### Add a Record or Row to the Grid

Figure 305 - The "+" sign opens a blank row at the top of the grid.

- Click the plus “+” sign to open a new blank row at the top of the grid.

- Enter all information across the row.

- When the entry is completed, exit the cell you are currently editing by clicking anywhere outside of the grid before saving the new or edited record by clicking on the “Disc” icon (shown below in Figure 294).

Figure 306 - The "Disc" icon saves a new record or edits an existing record.

Figure 307 - Use the "x" icon to cancel the new record.

- Cancel a new record by clicking on the “x.”

### Delete a Row to the Grid

Figure 308 - Select row for deletion by checking the box.

- Select the row for deletion by checking the box to the left of the row.

- Click the “Trash Can” icon.

- Click the “save” icon to complete deletion of the row.

### Actions and Settings at the Top of the Grid

#### Columns

- On the grid, columns can be either hid or viewed.

- To hide, turn off the “slider” for the respective column.

Figure 309- Slider column.

#### Filters

- Filters allow the user to search for specific records.

  - Select “Filter;” then the column.

  - Using the “Operation” dropdown menu, select contains, starts with or equals.

  - In the “Value” area, type in what the user wants to search for.

Figure 310 - Filters allow the user to search for records.

#### Density

- Density slightly changes the width and height of the information on the grid.

Figure 311 - Density slightly changes the height and width of the information on the grid.

#### Export

- Export typically downloads to a CSV file.

Figure 312 - Export is used to download a .csv file.

- The .csv file will download into the user’s workstation “Download” folder.

Figure 313 - .CSV file downloads to the Center Admin or users station download files.

- If the user sorted the grid record first, the sorted information would be the only information exported to the .csv file.

## Appendix III - Reports

### Available Reports

Reports will open into its own Tab. All the reports are generated in a similar process.

The key processes are:

- On the right side of the report panel are sortable selection. Such as:

  - Unit

  - Resource Types

  - Status

  - Use of Line up groups

  - Incident Types and Subtypes

- Date ranges are required except on:

- Morning Line Up Report

- In-Service Report

- Responding Resource List

- Open Incident

- On the left side of the report panel are the results from the selection process.

- Generate PDF will create the report based on the selection criteria.

### Morning Lineup Reports

Figure 314 – Example Morning Line Up Report

In Service Report

Figure 315 - In Service Report

Timer Report

Figure 316 – Timer Report

Timer Report - Added remaining columns from Timer panel to report page and generated PDF.

### Responding Resource List

Figure 317 - Responding Resource List

Single Resource Response History

Figure 318 – Single Resource Response History

### Open Incident

Figure 319 – Open Incident

### Incident By Date

Figure 320 – Incident by date.

Incident Numbers

Figure 321 – Incident Numbers

### Fire Report

Figure 322 – Fire Report

### Monthly Fire Summary

Figure 323 – Monthly Fire Summary

### Contract Report

Figure 324 – Contract Report

## Appendix IV – Incident Types

Table 2 - Incident Types

| Code | Description | Expired? |
| --- | --- | --- |
| A/C Down (expired 4/2024) | Aircraft Down | Yes |
| AC – Air Accident | Aircraft Accident | No |
| AC – Marine Accident | Marine Accident | No |
| AC – Motor Vehicle Accident | Motor Vehicle Accident | No |
| AC – Rail Accident | Rail Accident | No |
| AC – Structure Accident | Structure Accident | No |
| Aircraft | Aircraft | No |
| AP – Critical Incident Stress Management / Peer Support | Critical Incident Stress Management / Peer Support | No |
| AP – Law Enforcement (internal) | Law Enforcement | No |
| AP – Management Event (internal) | Management Event (internal) | No |
| AP – Resource Programs (internal) | Resource Programs (internal) | No |
| Emerg Stby | Emergency Standby | No |
| F1 – Debris /Product Fire | Debris Fire | No |
| F1 – Non-Statistical/Other | Nonstatistical Fire | No |
| F1 – Prescribed Fire | Prescribed Fire | No |
| F1 – Structure Fire | Structure Fire | No |
| F1 – Vehicle Fire | Vehicle Fire | No |
| F1 – Wildfire | Wildfire | No |
| FM – Complex Incident | Complex | No |
| FM – Emergency Stabilization | Emergency Stabilization | Yes |
| FM – False Alarm | False Alarm | No |
| FM – Fire Rehabilitation | Fire Rehabilitation | Yes |
| FM – Incident/Event Support | Incident/Event Support | No |
| FM – Out of Area Response | Out of Area Response | Yes |
| FM – Preparedness/Preposition | Preparedness/Preposition | No |
| Hazmat (expired 04/2024) | Hazmat | Yes |
| HZ – Biological or Toxic Conditions | Biological or Toxic Conditions | No |
| HZ – Explosives or Electrical Dangers | Explosives or Electrical Danger | No |
| HZ – Flammable as, Oil, and other liquid conditions | Flammable Gas | No |
| HZ – Radioactive/Nuclear Conditions | Radioactive/Nuclear Conditions | No |
| Med Aid | Medical Aid | No |
| Misc | Miscellaneous | No |
| MRO | Manage Res Obj | No |
| N/A | N/A | No |
| NatDisastr (expired 04/2024) | Natural Disaster | Yes |
| ND – Earthquake | Earthquake | No |
| ND – Flooding | Flooding | No |
| ND – Hurricane/Typhoon | Hurricane/Typhoon | No |
| ND – Landslide/Mass Earth Movement/ Avalanche/Sinkhole | Movement/Avalanche/Sinkhole | No |
| ND – Severe Winter Weather | Severe Winter Weather | No |
| ND – Thunderstorm/Tornado/High Winds | Thunderstorm/Tornado/High Winds | No |
| ND – Tsunami | Tsunami | No |
| ND – Volcano | Volcano | No |
| PA – Community Event | Community Event | No |
| PA – Infrastructure Event | Infrastructure Event | No |
| PA Public Service Organization Event | Public Service Organization Event | No |
| Pub Asst (expired 04/2024) | Public Assist | Yes |
| Resc Order | Resource Order | No |
| SAR (expired 04/2024) | Search & Rescue | Yes |
| Smoke Chk | Smoke Check | No |
| SR – Marine Search/Rescue/Recovery | Marine Search/Rescue/Recovery | No |
| SR – Medical Assist | Medical Assist | No |
| SR – Urban Search/Rescue/Recovery | Urban Search/Rescue/Recovery | No |
| SR – Wildland Search/Rescue/Recovery | Wildland Search/Rescue/Recovery | No |
| TR – Classroom Training | Classroom Training | No |
| TR – On-the-Job Training | On-the-Job Training | No |
| TR – Proficiency & Currency Event | Proficiency & Currency Event | No |
| TR – Simulation | Simulation | No |

## Appendix V – Other Data Zoom Levels

Table 3 – Other Data Zoom Levels

| Layer Name | Zoom Level | Display Order | Map Footer | Display Field |
| --- | --- | --- | --- | --- |
| NPS FMU | 9 | 10 | no | FMU_Code |
| BLM FMU | 13 | 11 | no | FMU_Code |
| USFS FMU | 13 | 12 | no | FMU_Code |
| RFPA | 13 | 13 | no | FMU_CODE |
| BIA FMU | 9 | 14 | no | FMU_Code |
| USFWS FMU | 13 | 15 | no | FMU_Code |
| Predictive Service Areas | 15 | 17 | yes | psanationalcode |
| Wilderness | 13 | 18 | yes | ShortName |
| Sage Grouse Habitat | 9 | 19 | no |  |
| IRWIN Incidents (last 30 days) | 9 | 20 | no |  |
| RAWS Stations | 9 | 101 | no | StationName |
| Surface Management | 13 | 102 | no | Label |
| Counties | 13 | 103 | yes | NAME |
| Lightning (last 2-7 days) | 7 | 104 | no |  |
| Lightning (last 24 hours) | 10 | 105 | no |  |
| Forest Service Topo | 1 | 106 | no |  |
| Military Training Routes | 9 | 107 | no |  |
| Temp Flight Restrictions (4) | 4 | 109 | no | EFFECTIVE |
| Public Land Survey | 13 | 110 | no |  |

## Appendix VI – Other Data URL Layers

Table 4 – Other Data URL Layers

| Layer Name | Layer URL |
| --- | --- |
| NPS FMU BLM FMU | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FireManagementUnits/MapServer/2/ |
| USFS FMU | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FireManagementUnits/MapServer/1/ |
| RFPA | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FireManagementUnits/MapServer/4/ |
| BIA FMU | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FireManagementUnits/MapServer/3/ |
| USFWS FMU | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FireManagementUnits/MapServer/0/ |
| Predictive Service Areas | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FireManagementUnits/MapServer/5/ |
| Wilderness | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_AgencyBoundaries/MapServer/0 |
| Sage Grouse Habitat | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USAWilderness/MapServer/0/ |
| IRWIN Incidents (last 30 days) | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/SageGrouse/MapServer/0/ |
| RAWS Stations | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/RAWS/MapServer/0/ |
| Surface Management | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_FederalLands/MapServer/0/ |
| Counties | https://services.arcgis.com/P3ePLMYs2RVChkJx/arcgis/rest/services/USA_Counties/FeatureServer/0/ |
| Lightning (last 2-7 days) | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/LightningStrikes/MapServer/2/ |
| Lightning (last 24 hours) | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/LightningStrikes/MapServer/1/ |
| Forest Service Topo | https://apps.fs.usda.gov/arcx/rest/services/EDW/EDW_FSTopo_01/MapServer/ |
| Military Training Routes | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/USFSFireCOP_MTR/MapServer/11 |
| Temp Flight Restrictions (4) | https://egp.wildfire.gov/arcgis/rest/services/FireCOP/TFRData/MapServer/0 |
| Public Land Survey | https://gis.blm.gov/arcgis/rest/services/Cadastral/BLM_Natl_PLSS_CadNSDI/MapServer/2/ |

# Part XI: Alternate Authentication (If FAMAuth is Unavailable)

The Alternate Authentication feature enables users to log into WildCAD-E using a secure alternative method. If FAMAuth is experiencing an outage and is unavailable, users are automatically redirected to this feature when attempting to log into WildCAD-E.

If FAMAuth is unavailable, the user can access WildCAD-E by going directly to the URL associated with the desired environment:

- WildCAD-E OAT is accessible at:  wildcadoat.firenet.gov.

- WildCAD-E PROD is accessible at:  wildcade.firenet.gov.

A user must have previously accessed at least one center in WildCAD-E via the FAMAuth method to successfully use the alternate authentication. After entering the URL for OAT or PROD and WildCAD-E detects that FAMAuth is unavailable, the user is prompted to generate a one-time secure code.

To start the process, click on “Generate Code.”

Figure 325 – Select Generate Code

The user is prompted to enter their email associated with their FAMAuth ID account for the code and link to log into WildCAD-E. Then, click on “Go.”

Figure 326 – Enter your e-mail.

Enter the user’s email. The code/link is only valid for two (2) minutes.

Figure 327– Go to your e-mail.

The user can click directly on the link in the email or copy the code and enter the code back at the WildCAD-E login page.

Figure 328– Email Link or Code.

Entering the code in the WildCAD-E login page, then click on “Go.”

Figure 329 – Using the Code.

Users will be logged into the last center or select a center with the same roles assigned as the last time you were working in that center.

Figure 330 – Select a center.

# Part XII: HelpDesk Contact Information

If a user needs technical support from WildCAD-E, go to the WildCAD-E Support site at: http://www.wildcadsupport.net/login.asp; enter user’s name and password; and complete the online form. Support personnel will respond as soon as the form is received.

WildCAD-E

User Guide for Dispatchers

V1.808.1, November 2024